Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	ENGINEERING THERMODYNAMICS				      	     Time : 3 hours

Subject Code:	12ME213										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Distinguish between open and closed system.

2.	List the limitations of first law of thermodynamics.

3.	State the Kelvin Planck statement of second law of thermodynamics.

4.	What is meant by available energy?

5.	What do you mean by Triple point?

6.	What is a pure substance? Give Examples.

7.	Distinguish between ideal and real gas.

8.	State the Avogadro’s law.

9.	Define Psychrometry.

10.	Define the term Degree of Saturation.

PART – B  (5 x 3 = 15 MARKS)

11.	State the difference between macroscopic and microscopic view points.

12.	One kg of air in a closed system initially at 5°C occupying a volume of 0.3m3 undergoes a         constant pressure heating process to 100°C. Find the change in entropy of air.

13.	A boiler generates steam at 10bar and dry saturated from water at 50°C.Calculate the heat        supplied to the boiler per kg of steam produced.

14.	What are the assumptions made for ideal gas equation?

15.	Draw the skeleton psychrometric chart and show the sensible heating and evaporative cooling        process on it.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Prove that the heat absorbed or rejected during a polytropic process is γ−nγ−1×workdone

Where γ is the ratio of specific heat and n is the polytropic index.            			    (7)

b.	Air occupies a space of 0.3m3 at a pressure of 2bar and a temperature of 80°C. It is heated at constant volume, until the pressure is 8bar. Determine

i.	Temperature at the end of the process 													ii.     Mass of air

iii.	Change in internal energy	   															For air Cv=0.718, Cp=1.005 and R=0.287 kJ/kgK.                                 				    (8)

(OR)

17.	In a steady flow apparatus, 135 kJ of work is done by each kg of fluid. The specific volume of the fluid, pressure and velocity at the inlet are 0.37m3/kg, 600kpa and 16m/s. The inlet is 32m above the floor and the discharge pipe is at floor level. The discharge conditions are 0.62m3/kg,100kpa and 270m/s. The total heat loss between the inlet and discharge is 9kJ/kg of fluid. In flowing through this apparatus, does the specific internal energy increase or decrease and by how much?

[P.T.O]

18.	a.	A heat pump is used to maintain an auditorium hall at 24°C when the atmospheric 	temperature is 10°C.The heat lost from the hall is 1500kJ/min.Calculate the power required 	to run the heat pump if its COP is 30% of Carnot machine working between the same 	temperature limits.                                                                                 				    (8)

b.	1130 kJ of heat is supplied to an engine from a source at 292°C.The engine rejects heat to a

sink at 12°C.Find which one of the following cycles represent a reversible, irreversible result  855kJ of heat is rejected 570Kj of heat is rejected. 							    (7)

(OR)

19. 	One kg of water at 273 K is brought into contact with a heat reservoir at 373 K. When the water has reached 373 K, find the entropy change of the water, of the reservoir and of the	universe. a.	If water is heated from 273 K to 373 K by first bringing it in contact with a reservoir 	at 	323K and then with a reservoir at 373 K , what will be the entropy change of the universe?

b.	Explain how water might be heated from 273K to 373K with almost no change in the 	entropy 	of the universe.

20.	Draw and explain the phase equilibrium diagram for a pure substance on h-s plot with relevant constant property lines.

(OR)

21.	Steam at 0.8 MPa, 250oC and flowing at the rate of 1 kg/s passes into a pipe carrying wet steam at 0.8 MPa, 0.95 dry. After adiabatic mixing the flow rate is 2.3 kg/s. Determine the condition of steam after mixing. The mixture is now expanded in a frictionless nozzle isentropically to a pressure of 0.4 MPa. Determine the velocity of the steam leaving the nozzle. Neglect the velocity of steam in the pipeline

22.	A Vessel of Volume 0.3m3 Contains 15 kg of air at 303 K. Determine the Pressure exerted

by the air using.

i.	Perfect Gas Equation.

ii.	Vander Waals Equation.

iii.	Generalized Compressibility chart.

Take Critical Temperature of air is 132.8 K and Critical Pressure of air is 37.7 bar.

(OR)

23.	a. 	Draw and explain the generalized compressibility chart.                                			    (9)

b. 	Air contains 21% of oxygen and 79% of nitrogen by volume. Determine i) molecular 	weight of air ii) Gas constant iii) Density of air at STP.                                                     (6)

24.	The sling psychrometer reads 40°C DBT and 28°C WBT. Calculate the following a) Specific humidity b) Relative humidity  c) Vapour density in air  d) Dew point temperature e) Enthalpy of the mixture. Assume atmospheric pressure to be 1.01325 bar.

(OR)

25.	40m3 of air at 35°C DBT and 50%RH is cooled to 25°C DBT maintaining its specific humidity constant. Without the aid of psychrometric chart. Calculate a) Relative humidity of cooled air b) Heat removed from air.