Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	THERMAL ENGINEERING I						      	     Time : 3 hours

Subject Code:	12ME214						 				       Maximum Marks: 100

#### Answer ALL questions

(Use of standard thermodynamic tables, Mollier diagram, Psychrometric chart and Refrigerant property tables are permitted).

PART – A  (10 x 1 = 10 MARKS)

1.	Steam generator is also called as _________.

2.	Cochran boiler is an example of water tube boiler (True or False).

3.	In a nozzle _________ of steam increases.

4.	Portion of nozzle at which the area is constant is called _________.

5.	Function of a turbine is to produce mechanical energy (True or False).

6.	Parson turbine is an example for _________ turbine.

7.	Piston cylinder arrangement is present in _________ type of compressor.

8.	Stroke volume is also called as _________ volume.

9.	Refrigerant used in aqua ammonia absorption refrigeration system is _________.

10.	The condition of refrigerant at the outlet of evaporator in a vapour compression cycle is saturated _________.

PART – B  (5 x 3 = 15 MARKS)

11.	List out the losses in a boiler.

12.	Draw a convergent divergent nozzle.

13.	What do you mean by stage in a turbine?

14.	Define volumetric efficiency of a compressor.

15.	Draw the temperature entropy diagram of vapour compression cycle in which the condition of refrigerant at the inlet to the compressor is in two phase and saturated at the outlet of the compressor.

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat sketch, explain the working principle of a Cochran boiler.

(OR)

17.	Discuss about the working principle of a Babcock and Wilcox boiler.

18.	Derive an expression for the pressure ratio for a gas flow in a nozzle.

(OR)

19.	Helium at 8 bar and 375 K expands isentropically in a convergent divergent nozzle into a space at 4 bar. Find the mass flow rate of helium if the exit area of the nozzle is 1 m2 and the initial velocity is zero. Assume Cp = 5.2 kJ/kgK and R = 2079 J/kgK.

20.	With a neat sketch, explain the pressure compounding of an impulse turbine.

(OR)

21.	Differentiate between impulse and reaction turbine.

[P.T.O]

22.	Derive an expression for work done per kg of air delivered by a single stage reciprocating air compressor without clearance.

(OR)

23.	Air is to be isentropically compressed at the rate 1 m3/s from 1 bar and 293 K to 10 bar. Find out the work of compression and the volumetric efficiency if the clearance volume is 4% of the stroke volume for single stage, two stage and three stage compression. Assume density of air to be 1.2 kg/m3.

24.	With a neat sketch, explain the working principle of a water-lithium bromide vapour absorption cooling system.

(OR)

25.	With the help of schematic, temperature entropy and pressure enthalpy diagram discuss the working of a vapour compression cycle.