Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	THERMAL ENGINEERING II						      	     Time : 3 hours

Subject Code:	12ME215						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In a four stroke engine, the working cycle is completed in __________ revolution of the crank shaft.

2.	The knocking tendency in spark ignition engines may be decreased by __________.

3.	The thermodynamic cycle on which the petrol engine work is__________.

4.	A diesel engine has __________ number of valves.

5.	Define Mach number.

6.	A convergent nozzle is said to be choked when the critical pressure is attained at the __________.

7.	Give the classifications of gas turbines.

8.	Classify rocket engines.

9.	Give the classifications of air conditioning systems.

10.	List few applications of air conditioning.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Mechanical efficiency and indicated thermal efficiency.

12.	Plot the P-V and T-S diagram of Dual cycle.

13.	Define the term Fanno flow.

14.	List the uses of gas turbines.

15.	Define ton of refrigeration.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain air cooling system and water cooling system for I.C engine with neat sketches.

(OR)

17.	Briefly explain with a neat sketch the operation of a simple float type carburater.

18.	The minimum pressure and temperature in an Otto cycle are 100kPa and 27ºC. The amount of heat added to the air per cycle is 1500kJ/kg. Determine the pressure and temperature at all points of the air standard Otto cycle. Also calculate the specific work and thermal efficiency of the cycle for a comparison ratio of 8:1.Take for air CV = 0.72kJ/kgk and γ = 1.4

(OR)

19.	Explain briefly the diesel cycle with the help P-V and T-S diagrams and derive an expression for the ideal efficiency of a diesel cycle.

20.	A gas at a pressure of 70 Kpa and temperature of 295 K enters a combustion chamber at a velocity of 75 m / sec. The heat supplied in a combustion chamber is 1250 KJ / Kg. Determine the Mach number, pressure and temperature of gas at exit. Take γ=1.3 and  R = 0.46 KJ / Kg K.

(OR)

[P.T.O]

21.	A conical air diffuser has an inlet diameter of 40cm and exit diameter of 80cm. Air enters the diffuser with a static pressure of 200KPa, static temperature of 37oC.The average velocity of flow at the inlet of the diffuser is 265m/s. Calculate a) mass flow rate b) the properties at the exit section and c) force exerted on the diffuser walls.

22.	Explain with neat sketches the reheating and regeneration method of improving the thermal efficiency of open cycle gas turbine plant.

(OR)

23.	Explain with a neat sketch explain the principle of operation of a turbojet engine and state its advantages and disadvantages.

24.	Explain any three types of air conditioning components with neat sketch.

(OR)

25.	Explain All – air high velocity system of air conditioning.