Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	MICROPROCESSOR AND MICROCONTROLLERS	    Time : 3 hours

Subject Code:	12MT217								 	             Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the size of address lines in  8085 microprocessor?

2.	How many flags are available in 8085 microprocessors?

3.	What is the function of TXRDY signal in 8251?

4.	Name the ports present in 8255 Programmable peripheral interface.

5.	What is the clock frequency of 8051 microcontroller?

6.	Mention the size of timer registers.

7.	Write an instruction to stop the execution in 8051 microcontroller.

8.	Explain the instruction DJNZ  1200.

9.	Which register in ARM processor is used as program counter?

10.	What is the size of general purpose register in ARM processor?

PART – B  (5 x 3 = 15 MARKS)

11.	List the interrupts in 8085 microprocessor.

12.	Write notes on RS232 bus.

13.	Draw the frame format of IE register.

14.	Compare JMP and CALL instructions.

15.	Write any three data movement instructions.

PART – C  (5 x 15 = 75 MARKS)

16.	With the architecture, explain the features of  8085 microprocessor.

(OR)

17.	Explain  the different types of addressing modes in 8085 microprocessor with two 	examples.

18.	Draw the block diagram of 8279 keyboard display interface and explain the functions of 	each block.

(OR)

19.	With the block diagram, discuss the different mode of operation of 8255 programmable peripheral interface.

20.	Discuss the features of 8051 microcontroller with a block diagram.

(OR)

21.	Describe the different modes of timers in 8051 with relevant diagram.

22.	a.	With an example explain any four Arithmetic instructions in 8051 microcontroller.	    (8)

b.	Find the addressing modes and explain of the following instructions.				    (7)

MOV  A,  B

MOV  A, #02

MOVX  DPTR, @A

(OR)

[P.T.O]

23.	a.	Write an Assembly language program to

i . 	Add two 8 bit numbers.

ii.  	Multiply two 8 bit numbers.												    (9)

b.	Discuss any three the Boolean variable manipulation instructions available in 8051 microcontroller with example.													    (6)

24.	a.	List the features of ARM processors with relevant diagram.						    (9)

b.	Write   the instructions involved in shift mechanism.								    (6)

(OR)

25.	a.	Draw the structure of ARM   registers in detail.									    (9)

b.	With an example explain any three branch instruction.							    (6)