Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	THIN FILMS TECHNOLOGY FOR ENGINEERS		     Time : 3 hours

Subject Code:	12PH209								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Insulators cannot be deposited by ___________ sputtering.

2.	Which pump is based on the principle of displacement of gap?

3.	In MBE system ___________ prevent re-evaporation from parts other than the hot cells.

4.	Which gauge is also called ionization gauge?

5.	What type of stress is applied to the film due to differential thermal expansion of the substrate?

6.	Adsorption that occurs by forming a covalent or ionic bond with a surface atom is known   as

___________.

7.	Debye Scherrer formula for particle size determination is given by D = ___________.

8.	A material in which the crystallites are nearly aligned in all 3 dimensions is called a ___________.

9.	___________ coatings are used to avoid the incoming sunlight from being reflected back off the solar cell.

10.	Gas sensor is a sub class of ___________ sensors.

PART – B  (5 x 3 = 15 MARKS)

11.	Mention the advantages and drawbacks of diffusion pump.

12.	Mention the different steps involved in sol-gel process.

13.	What is LASER ablation and what are its benefits?

14.	Compare polycrystalline, amorphous and single crystalline material based on texture.

15.	Explain the functioning of a gas sensor.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain with suitable diagram the instrumentation, working principle, advantages and drawbacks of turbo molecular pump.

(OR)

17.	With a neat sketch explain in detail the construction and working of ionization gauge.

18.	With suitable diagram explain the construction and working of molecular beam epitaxy.

(OR)

19.	With a neat sketch, explain the principle, kinetics and working concept of CVD.

20.	With appropriate figures, explain in detail, the different types of lattice mismatch.

(OR)

21.	With a neat sketch, explain in detail the various stages in thin film growth.

22.	Define Hall Effect and describe how this technique can be used to find the electrical properties of thin films.

(OR)

[P.T.O]

23.	Describe in detail the basic parts and working of UV-Visible spectrophotometer. Explain how to determine the transmittance and absorbance of thin films with suitable equations.

24.	a.	Discuss in detail the different structures of thin film transistor and the various steps 	involved in fabrication of thin film transistors.									  (10)

b.	Briefly discuss the steps in fabrication of a thin film solar cell.						    (5)

(OR)

25.	a.	What is a thin film diode? Explain in detail, the design and fabrication of thin film diode.

(10)

b.	Outline the basic steps in fabrication of MEMS device.							    (5)