Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	HEAT TRANSFER OPERATIONS					      	     Time : 3 hours

Subject Code:	12BT208										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Conduction is defined as _________.

2.	State law of conservation of mass.

3.	What do you mean by convection?

4.	Define radiation.

5.	What is LMTD?

6.	Draw a diagram of a counter flow heat exchanger.

7.	What is evaporation?

8.	As temperature increases rate of evaporation __________.

9.	What do you mean by boiling?

10.	Condensation means _________.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain law of conservation of energy.

12.	Name different types of convection.

13.	Write the LMTD expression for Heat Exchanger.

14.	Write short notes on evaporation process.

15.	Write a brief note on condensation.

PART – C  (5 x 15 = 75 MARKS)

16.	Write a detailed note on various laws of heat transfer phenomenon.

(OR)

17.	A furnace wall consists of two layers, 22.5cm of fire brick(k=1.2kcal/hr m oC)and 12.5cm of insulating brick (k=0.15kcal/hr m °C) . The temperature inside the furnace is 1650 oC and the inside heat transfer coefficient is 60kcal/hr m oC. The temperature of the surrounding atmosphere is 27C and the outside heat transfer coefficient is 10kcal/hr m2 C. Determine the rate of heat of loss per square meter of the wall.

18.	Water heated to 80C flows through a 2.54cm I.D and 2.88cm O.D steel tube ( k = 50 W/m K). The tube is exposed to an environment which is known to provide an average convection coefficient of ho = 30800 W/m2 K on the outside of the tube. The water velocity is 50 cm/s. Calculate the overall heat transfer coefficient based on the outer area of the pipe. Properties of water at bulk mean temperature of 80 C: =974 kg/m3 ,  = 0.364 x 10- 6 m2 /sec, k= 668.7 x 10- 3 W/m K, Pr = 2.20.

(OR)

19.	Derive the expression for dimensional analysis for liquid flowing through pipeline by laminar flow.

[P.T.O]

20.	Write a detailed note on Parallel and counter flow heat exchangers.

(OR)

21.	What do you mean by LMTD? Write the significance of LMTD in parallel flow.

22.	Derive the expression for single effect evaporator.

(OR)

23.	10,000 kg/hr of an aqueous feed containing 1% dissolved solids is to be concentrated to 20% 	solids, in a single effect evaporator. The feed enters at 25C. The steam chest is fed with 	saturated steam at 110C. The absolute pressure maintained in the evaporator is such that the 	water will boil at 55C. The boiling point elevation are as follows:

Feed: 0.2C

20% solution: 15C

The overall heat transfer coefficient, under normal operating conditions would be 2500 W/m2C. 	Estimate the steam requirement assuming no sub cooling of condensate, heat load on the 	condenser, and the heat transfer area.

24.	Explain in detail various types of boiling with neat diagram.

(OR)

25.	Elaborately explain various types of condensation in detail.