Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	09EE202/12EE220/EE257		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A digital circuit is also referred as a logic circuit. (True/False).

2.	Convert (100111010)2 to octal.

3.	List universal logic gates.

4.	Draw a half-adder truth table.

5.	A 4-bit synchronous counter uses flip-flops with propagation delay times of 15 ns each. The maximum possible time required for change of state will be ___________.

6.	How many flip flops are required to construct a decade counter?

7.	A table showing present states, next states and conditions at flip-flop is called ___________.

8.	Differentiate a counter and a shift register.

9.	What is meant by fan-out?

10.	A PAL can be used as a dynamic memory. (True/False)

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate weighted and non-weighted code with example.

12.	Implement X=AB+CD using only NAND gates.

13.	Compare combinational and sequential circuits.

14.	Which sequential circuits are difficult to design and why?

15.	Brief about EEPROM.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Convert (8A9D)16to binary, decimal and octal number.							    (6)

b.	Draw the logic circuit for the following equations. Simplify the equations and draw the simplified circuits.

i.	X= (B+D)(A+C) +ABD

ii.	YA⋅B		Y = 									    (9)

(OR)

17.	Simplify the following four variable switching function using Quine-McClusky tabulation method and Karnaugh map.

f(A, B, C, D) = Σ (0, 1, 2, 3, 4, 6, 8, 9, 10, 11)

18.	Design a 4-bit binary-to-gray code converter.

(OR)

19.	a.	Explain 4x1 multiplexer and 1x4 demultiplexer using logic diagrams.				    (8)

b.	Write short notes on encoder and decoder.										    (7)

20.	Define flip-flop. Illustrate the operation of four basic types of flip-flop with its truth table.

(OR)

[P.T.O]

21.	a.	Design a 3-bit Johnson counter using D-Flip flop.								    (8)

b.	Using JK flip-flop design an asynchronous decade counter.						    (7)

22.	Design a counter with the irregular binary count sequence shown in the state diagram below using JK flip-flops.

001

010

101

111

(OR)

23.	Explain the basic model and concept of sequential machines. Draw the state transition diagram of MOD-5 counter.

24.	Discuss on the basic elements in Field-Programmable Gate Array (FPGA).

(OR)

25.	With neat diagram, describe the basic operation of CMOS inverter circuit and ECL OR/NOR logic circuit.