Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	COMPUTER NETWORKS							      	     Time : 3 hours

Subject Code:	14CS2007										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List any two physical media used for data transmission in the Internet.

2.	In ______________ networks, the resources needed along a path (buffers, link transmission rate) to provide for communication between the end systems are reserved for the duration of the communication session.

3.	The World Wide Web’s application layer protocols, HTTP and HTTPS use port numbers ______________ and ______________respectively.

4.	Name any two FTP commands.

5.	What is full-duplex service?

6.	______________ is a transport layer protocol, which provides an unreliable, connectionless service to the invoking application.

7.	List the services provided by the network layer.

8.	Expand CIDR and NAT.

9.	Name the three categories of multiple access protocols.

10.	The protocol which translates IP addresses to MAC addresses is ______________.

PART – B  (5 x 3 = 15 MARKS)

11.	Consider a single router which is transmitting packets, each of length 16000 bits, over a single link with transmission rate 10 Mbps to another router at the other end of the link. Calculate the transmission delay.

12.	What is a socket?

13.	What is multiplexing and demultiplexing in transport layer?

14.	What is the use of TTL field in an IP datagram?

15.	Sketch the Ethernet frame structure.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the five-layer Internet protocol stack.

(OR)

17.	Describe in detail about Network Core.

18.	With the help of a diagram, explain in detail the working of Electronic Mail in the Internet.

(OR)

19.	a.	What are the services provided by DNS? Explain. 								    (8)

b.	Write short notes on the working principles of DNS.								    (7)

20.	a.	Discuss about connection establishment and connection release in TCP. 			  (10)

b.	Describe UDP segment structure with the help of a diagram.						    (5)

(OR)

21.	What are the various congestion control mechanisms in TCP? Explain.

22.	Explain about virtual circuit and datagram networks in detail.

(OR)

23.	Discuss in detail the internal components of a router.

24.	Write notes on Error Detection and Correction techniques.

(OR)

25.	Explain in detail about the infrastructure for network management.