Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2016

Subject Title :	FUNDAMENTALS OF ELECTRONICS				      	     Time : 3 hours

Subject Code:	14EC2090										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define doping.

2.	What is the barrier potential of germanium?

3.	How many pins does an op amp have?

4.	Draw the symbol of p channel JFET.

5.	Draw the truth table of EX-OR gate.

6.	What is the output of NOR gate, when both the inputs are high?

7.	_____________ is a non volatile memory.

8.	List out one source for noise.

9.	_____________ Satellite receives amplifies and retransmits the signal back towards earth.

10.	What is the intermediate frequency in super heterodyne receiver?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the energy band diagram for a conductor.

12.	What are the ideal properties of op amp?

13.	Draw the logic diagram, symbol and equation of a NAND gate.

14.	Define semiconductor memory and list out its properties.

15.	Define modulation. Point out the significance of modulation in a communication system.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With neat sketch explain the characteristics, construction and working principle of PN 	diode. 																		  (10)

b.	Explain in detail about p type and n type semiconductor.							    (5)

(OR)

17.	a.	Discuss the Common base configuration of BJT and plot the dc characteristics.		  (10)

b.	Explain the internal architecture of op amp with neat block diagram.				    (5)

18.	a.	List out the logic gates with the logic symbol, logic table and logic equation.		  (10)

b.	(875.12)10 = (  )8   																    (3)

c.	(1110101)2  = (  )16  															    (2)

(OR)

19.	Explain the integrator circuit with a neat diagram. How will you overcome the problems of basic integrator circuit?

20.	a.	What are the different types of memory? Discuss them in detail.					  (12)

b.	Explain briefly about the telemetry system.										    (3)

(OR)

21.	Explain the architecture of 8085 micro processor in detail with neat diagram.

[P.T.O]

22.	Define amplitude modulation. Derive the modulated signal of frequency modulation.

(OR)

23.	Explain the working of LVDT in detail with neat diagrams

24.	Explain the principle of super heterodyne receiver with neat block diagram.

(OR)

25.	a.	Draw and explain the basic block diagram of communication system.				  (10)

b.	List out the applications of MEMS and Nano electronics.							    (5)