Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14AE2017 | Duration : | 3hrs |
| Sub. Name : | AIRCRAFT PROPULSION | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Briefly explain the factors that effect the rate of reaction. | CO2 | 20 |
| (OR) |  |  |  |  |
| 2. |  | A Bryton cycle works between 1 bar, 300 K and 5 bar, 1250 K. There are two stages of compression with perfect intercooling and two stages of expansion. The work out of first expansion stage being used to drive the two compressors, where the interstage pressure is optimized for the compressor. The air from 1st stage is being heated to 1250 K and expanded. Calculate the power output of the free power turbine and cycle efficiency without and with a perfect heat exchanger and compare them. Also calculate the percentage improvement in the efficiency because of addition of heat exchangers. | CO1 | 20 |
| 3. | a. | What are the different types of aviation fuel ? | CO2 | 7 |
|  | b. | Explain the desired properties of the aviation fuel also state their limitation. | CO2 | 13 |
| (OR) |  |  |  |  |
| 4. |  | Determine the number of radial impeller vanes using Stanitz formulae for a centrifugal compressor which requires 180 kJ of power input per unit mass flow rate and is running at 15000 rpm. Guide vanes at inlet gives the air a prewhirl of 25° to the axial direction at all radii and the mean dia of eye is 250 mm. Impeller tip dia is 600 mm. The absolute air velocity at inlet is 150 m/s. | CO1 | 20 |
| 5. | a. | What are the losses in a centrifugal compressor? | CO2 | 6 |
|  | b. | Explain the phenomenon of surging and choking with a neat sketch | CO2 | 14 |
| (OR) |  |  |  |  |
| 6. |  | In a turbojet unit with forward facing ram intake, the jet velocity relative to the propelling nozzle at the exit is twice the flight velocity. Determine the rate of fuel consumption in kg/s, when developing a thrust of 25000N under the following conditions Ambient pressure and temperature : 0.7 bar; 1°C Compression total ead pressure ratio : 5:1 Flight speed : 800 km/h CV of fuel : 42000 kJ/kg Ram efficiency : 100% Isentropic efficiency of the compressor : 85% Isentropic efficiency of the of turbine : 90% Combustion efficiency : 98% Turbine pressure ratio : 2.23 Assume the mass flow of fuel is small compared with the mass flow of air and the working fluid throughout has the properties of air at low temperature. Neglect the extraneous pressure drop.Assume Cpg = Cpa = 1.005 kJ/kg K. | CO1 | 20 |
| 7. | a. | What are After burner? | CO2 | 2 |
|  | b. | List the different methods used to operate an After burner. | CO2 | 6 |
|  | c. | Explain the operation, process and its characteristics. | CO2 | 12 |
| (OR) |  |  |  |  |
| 8. |  | In a single stage impulse turbine the nozzle discharges the fluid on the blades at an angle of 60 ° to the axial direction and the fluid leaves the blade with an absolute velocity of 310 m/s at an angle of 30° to the axial direction. If the blades have equal inlet and outlet angles and there is no axial thrust, estimate the blade angles, power produced per kg/s of the fluid and the blade efficiency. | CO1 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | With a neat sketch explain the different components of an aircraft engine. | CO2 | 16 |
|  | b. | Define work cycle? | CO2 | 4 |

ALL THE BEST