Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14AE3001 | Duration : | 3hrs |
| Sub. Name : | ADVANCED SOLID MECHANICS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | At a point in a strained material the principal stresses are 100 N/mm2 (tensile) and 60 N/mm2 (compressive). Determine the normal stress, shear stress and resultant stress on a plane inclined at 50˚ to the axis of major principal stress and also determine the maximum shear stress at the point analytically and grapphically. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | Derive an expression for the major and minor principal stresses on an oblique plane, when the body is subjected to direct stresses in two mutually perpendicular directions accompanied by a shear stress. | CO1 | 15 |
|  | b. | Find the diameter of a circular bar which is subjected to an axial pull of 160kN,if the maximum allowable shear stress on any section is 65 N/mm2 . | CO1 | 5 |
| 3. | a. | A rolled steel joist of T section has the dimensions as shown in fig. below. The beam of T section carries a uniformly distributed load of 40kN/m run on a span of 10 m, calculate the maximum stress produced due to bending. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Derive an expression for bending stress at a layer in abeam. | CO1 | 10 |
|  | b. | A rectangular beam 200 mm deep and 300 mm wide is simply supported over a span of 8 m. What uniformly distributed load per metre the beam can carry,if the bending stress is not to exceed 120 N/mm2. | CO1 | 10 |
| 5. |  | Derive the expression of contact stresses with a suitable example. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Determine the shear stress distribution for a contact pressure distribution of p(y1)=p0 in the region -a≤y≤a as shown in fig. below. | CO2 | 10 |
|  | b. | Obtain the expression of maximum contact pressure and principal stresses for spherical contact.of maximum contact pressure and principal stresses in the casnd shear stress if the roller is in line cont | CO2 | 10 |
| 7. |  | Derive the expression for stresses in a plate with an elliptical hole. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Determine the surface of contact and the maximum pressure between two circular cyclinders whose axes are mutually perpendicular. | CO2 | 10 |
|  | b. | Derive the expression for stress and displacement in the beam resting on a continuous foundation. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Determine the shear center of the given channel section as shown in fig. below due to a shear load Sy of 300 kN applied through the shear centre of the section. 40cmm | CO2 | 20 |

ALL THE BEST