Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14CE3002 | Duration : | 3hrs |
| Sub. Name : | Advanced Concrete Technology | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Cement after setting shall not undergo any appreciable change in volume. Explain how this property of cement is evaluated using Le .Chatelier apparatus with neat sketch. | CO1 | 5 |
|  | b. | ‘Manufacturing processes of Portland cement determines its quality’ – discuss the manufacturing processes with neat sketches. | CO1 | 10 |
|  | c. | Differentiate workability agent and retarders. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Heavy weight aggregates are used for shielding and to produce high density concrete List the names of any two heavy weight aggregates. | CO1 | 4 |
|  | b. | Explain the lab test and field test on cement with the aid of diagrams wherever applicable. | CO1 | 12 |
|  | c. | Nowadays PPC is widely used for construction. Briefly explain PPC and its advantages over OPC. | CO1 | 4 |
| 3. | a. | Discuss the factors helping concrete to have more lubricating effect to reduce internal friction for helping easy compaction in detail | CO1 | 7 |
|  | b. | Apart from the report of ACI committee 212, M.R.Rixom also classified the admixture. Explain them in detail with respect to their application. | CO1 | 7 |
|  | c. | How does the shape and size of aggregates help in modifying the property of concrete. | CO1 | 6 |
| (OR) |  |  |  |  |
| 4. | a. | As a quality engineer in Park construction,, you are assigned to check the quality in the concrete manufactured in site. Elaborate on the step by step process of manufacturing with its significance. | CO1 | 10 |
|  | b. | State the precise and sensitive workability test done by measuring density ratio. Define the same. Explain the method with the aid of sketches. | CO1 | 10 |
| 5. | a. | Explain the term modulus of elasticity of concrete. Mention how it is experimentally determined as per IS code? | CO1 | 10 |
|  | b. | The strength of a fully matured concrete is found to be 30 MPa. Find the strength of an identical concrete at the age of 14 days when cured at an average temperature during day time at 22° C and night time at 8° C. | CO1 | 4 |
|  | c. | Explain the methods of curing of concrete. | CO1 | 6 |
| (OR) |  |  |  |  |
| 6. | a. | Design a concrete mix as per IS with the following data: Characteristic compressive strength…………………….. 30 N/mm2 Maximum size of coarse aggregate…………………….. 20mm Degree of workability……………………………100 mm(slump value) Degree of control………………………………………. Good Type of exposure……………………………………… Severe Specific gravity of cement…………………………….. 3.15 Specific gravity of coarse aggregate………………….. 2.60 Specific gravity of fine aggregate………………….. …. 2.60 Water absorption – Coarse aggregate…………………. 0.5% Water absorption – Fine aggregate……………………. 1.0% Free surface moisture – fine aggregate………………… 0% Entrapped air…………………………………………… 0% Fine aggregate grading………………………………….. Zone III Assuming suitable datas wherever necessary calculate the quantity of materials required to cast slab of area 10,000sft and 4 ½” thick. | CO1 | 12 |
|  | b. | Explain the concept of mis design. | CO2 | 2 |
|  | c. | What are the variables and various methods of proportioning. | CO2 | 3 |
|  | d. | Give the relationship between average design strength and specified minimum strength. | CO2 | 3 |
| 7. | a. | Formulate the relationship between compressive strength and tensile strength. | CO1 | 4 |
|  | b. | Explain the rheological approach of creep. | CO1 | 4 |
|  | c. | Discuss plastic shrinkage and drying shrinkage. | CO1 | 4 |
|  | d. | Define durability of concrete and its significance. | CO1 | 4 |
|  | e. | Write a note on the impact of water cement ration on durability of concrete. | CO1 | 4 |
| (OR) |  |  |  |  |
| 8. |  | Write a note on a)shotcrete b) vaccum concrete c) ferrocement d)No fines concrete. | CO1 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | How to evaluate an old monument or structure for their durability without disturbing a part or whole. Give the experimental procedure for any five tests with relevant skethches. | CO1 | 20 |