Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14CE3006 | Duration : | 3hrs |
| Sub. Name : | FINITE ELEMENT METHODS IN ENGINEERING | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Derive the stiffness matrix for 1D Truss element | CO1 | 15 |
|  | b. | List the properties of stiffness matrix. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | With flow chart and example, explain the basic steps of FEA | CO1 | 15 |
|  | b. | Write the applications of Finite element analysis in Mechanical engineering | CO1 | 5 |
| 3. |  | Calculate the nodal displacements and nodal forces for the bar loaded as shown in the Figure1. 6kN 12kN/mm (K1) 8kN/mm (K2) Figure1 | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Derive the shape functions for 1D Beam element | CO1 | 20 |
| 5. |  | For the triangular element shown in Figure2, the nodal values of displacement are: u1 = 0.001; u2 = 0.003; u3 = -0.002 v1 = -0.004; v2 = 0.002; v3 = 0.005 Find the displacement u,v at point P (2, 5) within the element. (All Dimensions are in mm) u  (1,1) 1 2 (3,4) 3 (2, 7) v pu Figure 2 | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Derive the shape functions for 2D bilinear rectangular element (use local coordinate system) | CO1 | 20 |
| 7. |  | Consider the differential equation for the problem as, d2ydx2+300x2=0 ; 0 £x £ 1 with the boundary conditions y (0) = 0 and y(1) = 0. Find the solution of the problem using a one coefficient trial function as Y=a1X(1-X3). Use (i) Point collocation method (ii) sub domain method (iii) Least square method (iv) Galerkin’s method. | CO1 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Find the maximum deflection for a clamped beam subjected to uniformly distributed load throughout its length. Use point collocation method. Assume trial function as y=a(x5-2Lx4+L2x3) | CO1 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | A steel rod of diameter d=2cm, length 5cm and thermal conductivity k=50W/m0C is exposed at one end to a constant temperature of 320oC(shown in Figure3). The other end is in ambient air of temperature 20oC with a convective coefficient of h=100 W/m20C. Determine the temperature at the mid point of the rod. Figure 3 320C L=5 cm 2 cm diameter φf=20∘C, h=100Wattsm2 ∘C , k=50Wattsm ∘C | CO1 | 20 |

ALL THE BEST