Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14EC2029 | Duration : | 3hrs |
| Sub. Name : | EMBEDDED SYSTEM DESIGN | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Write an embedded c program to operate a DPDT relay connected with 8085 microprocessor. Draw the hardware block diagram and software flowchart. | CO3 | 12 |
|  | b. | Explain ICE with a neat diagram. And Compare Emulator with ICE | CO2 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | Write an embedded c program to rotate the stepper motor in forward and reverse direction using a transistor motor driver Circuit, interfaced with 8051 microcontroller. Draw the hardware block diagram and software flowchart. | CO3 | 12 |
|  | b. | Mention the purpose of the following (i) Linker iii) Locator (ii)Compiler iv) Loader | CO2 | 8 |
| 3. | a. | With a neat diagram elucidate the embedded design life cycle with an example | CO3 | 15 |
|  | b. | Explain the process flow for establishing the serial communication. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Analyze the hardware requirement for any embedded application along With the product Specification | CO1 | 15 |
|  | b. | Discuss the Issues in Hardware / Software design and Co-design | CO1 | 5 |
| 5. | a. | Design an interfacing circuit for a closed loop DC motor control | CO1 | 10 |
|  | b. | What is the need for IDE in an Embedded Architecture? Discuss with an example. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the skills required for an embedded system designer for developing small scale, medium scale and sophisticated system. | CO2 | 10 |
|  | b. | Define semaphore and explain binary semaphore and Mutex with relevant diagrams. | CO3 | 10 |
| 7. | a. | Design an embedded system which displays minutes and second. Give the software flowchart. | CO3 | 16 |
|  | b. | Explain SCI along with the control Signals. | CO1 | 4 |
| (OR) |  |  |  |  |
| 8. | a. | Design an embedded system for an Automatic Washing machine using RTOS. Explain the different task and give the hardware block diagram. | CO1 | 14 |
|  | b. | Sketch the hardware block diagram of Bio Metric Attendance reader system. | CO1 | 6 |
|  |  | Compulsory: |  |  |
| 9. | a. | Design an embedded system for an Automatic Coffee Vending Machine using RTOS. Explain the different task and give the hardware block diagram. | CO1 | 14 |
|  | b. | Mention the importance of scheduling in the RTOS system. | CO3 | 6 |

ALL THE BEST