Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14EC3019 | Duration : | 3hrs |
| Sub. Name : | DIGITAL SYSTEM AND ASIC DESIGN | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Implement the following Boolean functions using the PAL device. W(A, B, C, D) = Σm(2, 12, 13) X(A, B, C, D) = Σm(7, 8, 9, 10, 11, 12, 13, 14, 15) Y(A, B, C, D) = Σm(0, 2, 3, 4, 5, 6, 7, 8, 10, 11, 15) Z(A, B, C, D) = Σm(1, 2, 8, 12, 13) | CO1 | 10 |
|  | b. | Draw the ASM chart for a mod -8 binary up- down counter. | CO1 | 6 |
|  | c. | Compare Mealy machine and Moore machine. | CO1 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | Design a Asynchronous sequential circuit with two inputs x1 and x2 and one output Z. Initially, both inputs are equal to zero. When x1 or x2 becomes ‘1’, the output Z becomes 1. When the second input also becomes 1, the output changes to 0. The output stays at 0 until the circuit goes back to the intial state. Draw the Asynchronous sequential logic circuit using D flipflop. | CO1 | 12 |
|  | b. | Explain the different types of Hazards? | CO1 | 3 |
|  | c. | Implement a Full adder circuit with a decoder and two OR gates S(x,y,z) = Σm (1,2,4,7) C(x,y,z) = Σm (3,5,6,7) | CO1 | 5 |
| 3. | a. | Design a Sequence Detector that produces output ‘1’ using Mealy machine whenever the overlapping sequence 101101 is detected using ASM chart.Draw the Synchronous sequential logic circuit for the same using JK flipflop. | CO1 | 12 |
|  | b. | Draw the PLA realization for the above circuit(Sequence Detector). | CO1 | 6 |
|  | c. | Mention the advantages of PLD. | CO1 | 2 |
| (OR) |  |  |  |  |
| 4. | a. | What is Programmable ASIC? Mention the different programming technologies used in Programmable ASIC. | CO2 | 3 |
|  | b. | Explain the Xilinx XC 4000 family IOB architecture with neat diagram. And also explain the Xilinx LCA timing model with diagrams. | CO3 | 12 |
|  | c. | Explain the problems that occur in the Asynchronous Sequential circuits while designing? | CO1 | 5 |
| 5. | a. | List out the different types of ASIC? Explain briefly about Semi custom ASIC? | CO2 | 13 |
|  | b. | Draw the Altera FLEX architecture (Logic array Block) and explain its working? | CO3 | 7 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the Altera MAX architecture (Logic array Block) with neat diagram. And also explain the Altera MAX timing model with diagrams. | CO3 | 10 |
|  | b. | Draw the Physical Design flow and briefly explain the process flow with the significance of each stage in the design flow? | CO2 | 10 |
| 7. | a. | Explain the Xilinx LCA interconnect architecture with neat diagram? | CO3 | 12 |
|  | b. | List out the differences between the CPLD and FPGA. | CO3 | 2 |
|  | c. | Draw the Altera MAX 9000 interconnect scheme and explain it’s working. | CO3 | 6 |
| (OR) |  |  |  |  |
| 8. | a. | Draw the Xilinx FPGA Design flow and briefly explain the process flow with the significance of each stage in the design flow? | CO2 | 8 |
|  | b. | Explain the format of the input and output files used by the PLA design tool expresso. | CO2 | 4 |
|  | c. | Explain hierarchical nature of an EDIF (Electronic Design Interchange Format) file. | CO2 | 6 |
|  | d. | What is meant by Half gate ASIC? | CO2 | 2 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain Xilinx Spartan II Configurable logic block in detail. | CO3 | 8 |
|  | b. | Design BCD counter using CPLD structure. (Use D flipflop). | CO3 | 12 |