Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14EC3020 | Duration : | 3hrs |
| Sub. Name : | CMOS VLSI Design | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Calculate the threshold voltage VTO at VSB = 0, for a polysilicon gate n-channel MOS transistor, with the following parameters: substrate doping density NA=1016 cm-3, polysilicon gate doping density ND=2x1020 cm-3, gate oxide thickness tox=500Å, and oxide interface fixed charge density Nox = 4x1010cm-2 . | CO1 | 8 |
|  | b. | With Gradual Channel Approximation (GCA) method find the derivation of ID Cutoff, Non Saturation and Saturation region and draw the Drain voltage Vs Drain current characteristics. | CO1 | 12 |
| (OR) |  |  |  |  |
| 2. | a. | Examine the mechanisms of channel pinch-off and current flow in saturation mode and discuss about channel length modulation. | CO1 | 8 |
|  | b. | Discuss in detail about the Oxide related and Junction capacitances. | CO1 | 12 |
| 3. | a. | Explain the DC and transfer characteristics of CMOS inverter and derive the expression for output voltage in various operating regions. | CO1 | 12 |
|  | b. | Synthesis a complex CMOS gate whose function is F = D + A· (B +C). | CO1 | 8 |
| (OR) |  |  |  |  |
| 4. | a. | Explain the concepts involved in Robust and Efficient Pass-Transistor Design. | CO3 | 14 |
|  | b. | Consider 16 cascaded minimum sized transmission gates, each with an average resistance of 8k.The node capacitance consists of the capacitance of two NMOS and PMOS devices. The capacitance is 3.6fF.Find the propagation delay of a network of 16 transmission gates. | CO1 | 6 |
| 5. | a. | Explain about propagation delay of CMOS Gate and obtain the equivalent RC model for 2-input CMOS gate. | CO1 | 8 |
|  | b. | Explain in detail about the design techniques for large fan-in. | CO1 | 12 |
| (OR) |  |  |  |  |
| 6. | a. | Explain in detail the various signal integrity issues in Dynamic CMOS logic. | CO3 | 14 |
|  | b. | Draw the function Z= A.B+ C.D using Domino CMOS Logic. | CO3 | 6 |
| 7. | a. | Define ratioed logic and explain in detail about pseudo NMOS logic. | CO3 | 10 |
|  | b. | With neat diagram explain in detail about DCVSL and pass transitor logic. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Discuss in detail about cascading Dynamic CMOS logic. | CO3 | 7 |
|  | b. | Discuss in detail about Dynamic Transmission gate edge triggered and C2MOS master slave positive edge triggered register. | CO3 | 10 |
|  | c. | Draw the basic block of np-CMOS logic. | CO3 | 3 |
|  |  | Compulsory: |  |  |
| 9. | a. | Draw the stick diagram and layout of 2-input CMOS NOR Gate | CO2 | 5 |
|  | b. | Draw the basic block of 4X4 array multiplier. | CO3 | 5 |
|  | c. | With neat diagram explain in detail about carry by pass adder and also design 16-bit carry by pass adder. | CO3 | 10 |

ALL THE BEST