Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14EE2001 | Duration : | 3hrs |
| Sub. Name : | Electric circuits & Networks | Max. marks : | 100 |

| Q. No. | Questions |  |  |  | Course outcome | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| PART-A (40X1=40 MULTIPLE CHOICE QUESTIONS) |  |  |  |  |  |  |
| 1. | Which of the circuit element given below is an active element. |  |  |  | CO1 |  |
|  | a. Voltage Source | b. Resistor | c. Inductor | d. Capacitor |  | (1) |
| 2. | . ______________ consumes power. |  |  |  | CO1 |  |
|  | a. .Voltage Source | b. Resistor | c. Inductor | d. Capacitor |  | (1) |
| 3. | . ______________ stores energy in its electro static field. |  |  |  | CO1 |  |
|  | a. Voltage Source | b. Resistor | c. Inductor | d. Capacitor |  | (1) |
| 4. | If the power consumed in the circuit below is 8 watts, find the current through the circuit . |  |  |  | CO1 |  |
|  | a. 1 amps | b. 2 amps | c. 5 amps | d. ) 10 amps |  | (1) |
| 5. | . _________ law states that the algebraic sum of all the currents meeting at a junction is zero |  |  |  | CO1 |  |
|  | a. Faraday’s law | b. Ohms law | c. Kirchoff’s voltage law | d. Kirchoff’s current law |  | (1) |
| 6. | What is the value of “I” in the circuit given below |  |  |  | CO1 |  |
|  | a.1 amp | b. 2 amps | c. 5 amps | d. 10 amps |  | (1) |
| 7. | Find V in the circuit shown below. |  |  |  | CO1 |  |
|  | a. 2 V | b. 5 V | c. 6V | d. 10 V |  | (1) |
| 8. | Find RAB |  |  |  | CO1 |  |
|  | a. 2Ω | b.3Ω | c.4Ω | d.6Ω |  | (1) |
| 9. | Find RAB |  |  |  | CO1 |  |
|  | a.2Ω | b. 3Ω | c. 4Ω | d. 6Ω |  | (1) |
| 10. | Voltage across ________ will be in phase with current through it. |  |  |  | CO1 |  |
|  | a. Resistor | b. Inductor | c. Capacitor | d. All of the above |  | (1) |
| 11. | The operator ‘j’ is equivalent to ___________ |  |  |  | CO1 |  |
|  | a. 11800 | b. ). 1900 | c. 100 | d. None of the above |  | (1) |
| 12. | A 4Ω resistor, j4Ω inductive reactance and a j4Ω capacitive reactance are in series across a source voltage. What is the net impedance across the voltage source. |  |  |  | CO1 |  |
|  | a. 4Ω | b.(4+j4)Ω | c. (4-j4)Ω | d. (4+j8)Ω |  | (1) |
| 13. | Super mesh circuit analysis is done when there is a _________ in the common branch of two meshes. |  |  |  | CO1 |  |
|  | a. Ideal voltage source | b. Ideal current source | c. Practical voltage source | d. Practical current source |  | (1) |
| 14. | ______ theorem is applicable to a circuit only when there are more than one source present in the circuit. |  |  |  | CO1 |  |
|  | a. Superpostion | b. Thevinin’s | c. Norton’s | d. Maximum power transfer |  | (1) |
| 15. | In the circuit shown below, determine the value of RL at which maximum power will be transferred to the load . |  |  |  | CO1 |  |
|  | a. 1Ω | b.2Ω | c.5Ω | d.10Ω |  | (1) |
| 16. | When a series RLC circuit resonates, the supply voltage and the current through the circuit will be _____ . |  |  |  | CO2 |  |
|  | a. ). In phase | b. Out of phase by 900 | c. Out of phase by 1800 | d. None of the above |  | (1) |
| 17. | If the current through a series RLC circuit at resonance is I, current through the circuit at half power frequency will be __________. |  |  |  | CO2 |  |
|  | a. I | b. I2 | c. I2 | d. 2I |  | (1) |
| 18. | The equivalent inductance of the circuit shown below, Leq = _______ |  |  |  | CO1 |  |
|  | a. L1+L2 | b. L1 - L2 | c. L1+L2-2M | d. L1+L2+2M |  | (1) |
| 19. | In a 3 phase circuit with balanced delta connected load if the line voltage is 400V, ____ will be the phase voltage |  |  |  | CO1 |  |
|  | a. 400 V | b. 4003 | c. 400∗3 | d. None of the above |  | (1) |
| 20. | In the circuit shown below, what will be the phase current? |  |  |  | CO1 |  |
|  | a. 23.1Amps | b.40 Amps | c. 69.3 Amps | d.None of the above |  | (1) |

| 21. | The time constant of a series RL Circuit is __________ |  |  |  | CO2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | a. τ = L/R | b. τ = R2L | c. τ = RL | d. τ = R/L |  | (1) |
| 22. | In a Y-connected circuit, between each line voltage and the nearest phase voltage, there is a phase angle of |  |  |  | CO1 |  |
|  | a. 30° | b. 0° | c. 90° | d. 60° |  | (1) |
| 23. | In a three-phase system, when the loads are perfectly balanced, the neutral current is |  |  |  | CO1 |  |
|  | a. two-thirds of maximum | b. zero | c. one-third of maximum | d. at maximum |  | (1) |
| 24. | In a certain Y-Y system, the source phase currents each have a magnitude of 9 A. The magnitude of each load current for a balanced load condition is |  |  |  | CO1 |  |
|  | a. 3A | b. 12A | c. 9A | d. 27A |  | (1) |
| 25. | In a Y-connected circuit, the magnitude of each line current is |  |  |  | CO1 |  |
|  | a. Three times the corresponding phase current | b. one-third the phase current | c. Equal to the corresponding phase current | d. zero |  | (1) |
| 26. | If in a Y-connected ac generator, each phase voltage has a magnitude of 90 VRMS, what is the magnitude of each line voltage? |  |  |  | CO1 |  |
|  | a. 90 V | b.0 V | c. 156 V | d. 180 V |  | (1) |
| 27. | In two wattmeter method of power measurement if the total power is measured by one wattmeter only then P.F of the system is |  |  |  | CO1 |  |
|  | a. 0 | b. 0.5 | c. 1 | d. none of the above |  | (1) |
| 28. | In a three-phase system, the voltages are separated by |  |  |  | CO1 |  |
|  | a. 120° | b. 180° | c. 45° | d. 90° |  | (1) |
| 29. | A resonant circuit has a lower critical frequency of 7 kHz and an upper critical frequency of 13 kHz. The bandwidth of the circuit is |  |  |  | CO2 |  |
|  | a. 10KHz | b. 12KHz | c. 6KHz | d. 8KHz |  | (1) |
| 30. | In single tuned circuit there is only one capacitor on |  |  |  | CO2 |  |
|  | a. Primary side | b .secondary side | c. both side | d. none |  | (1) |
| 31. | What is the total impedance of a 60 Hz series RLC circuit when XL = 7.5 ohms, XC =265.3 ohms, and R = 33 ohms? |  |  |  | CO2 |  |
|  | a. 290.8 ohms | b. 257.8 ohms | c. 259.9 ohms | d. 1989.75 ohms |  | (1) |
| 32. | Time constant in R – C series circuit is ____________. |  |  |  | CO2 |  |
|  | a. C/R | b. R/C | c. RC | d. RC/2 |  | (1) |
| 33. | The pass band of a constant k filter with Z1 and Z2 as series and shunt arm impedances is given by |  |  |  | CO3 |  |
|  | a. | b. | c. | d. |  | (1) |
| 34. | A 47 resistor is in series with an inductive reactance of 120 ῼ across an ac source. The impedance, expressed in polar form, is |  |  |  | CO1 |  |
|  | a. 4768.6° ῼ | b. 12068.6° ῼ | c. 12931.4° ῼ | d. 12968.6° ῼ |  | (1) |
| 35. | What is the maximum value of co-efficient of coupling? |  |  |  | CO2 |  |
|  | a. 1.2 | b. 0.5 | c. 0.8 | d. 1.0 |  | (1) |
| 36. | A 2-port network using z-parameter representation is said to be reciprocal if |  |  |  | CO1 |  |
|  | a. Z11 = Z22 | b. Z12 = Z21 | c. Z12 = - Z21 | d. Z11 Z22 = - Z12Z21 = 1 |  | (1) |
| 37. | A ________ filter significantly attenuates all frequencies below fc and passes all frequencies above fc. |  |  |  | CO3 |  |
|  | a. low-pass | b. high-pass | c. band-pass | d. band-stop |  | (1) |
| 38. | For the network of figure KVL for first loop is |  |  |  | CO2 |  |
|  | a. | b. | c. | d. |  | (1) |
| 39. | In a parallel circuit containing set of bulbs, if one of the bulb blew up the effective resistance offered will: |  |  |  | CO1 |  |
|  | a. Remains same | b. Increase | c. Fluctuate | d. Decrease |  | (1) |
| 40. | If R1 is in series with parallel connected to R2 and R3, what happens to total current if R2 opens? |  |  |  | CO1 |  |
|  | a. Increases | b. remains the same | c. decreases | d. none of these |  | (1) |

| PART B(8 X 5 = 40 MARKS) (ANSWER ANY EIGHT) |  |  |  |
| --- | --- | --- | --- |
| 41. | Determine vo and i in the circuit shown in Figure. | CO1 | (5) |
| 42. | Find out the equivalent resistance between terminals A and B in the Figure. | CO1 | (5) |
| 43. | Use the superposition theorem to find v in the circuit in Figure . | CO1 | (5) |
| 44. | State and illustrate Norton’s theorem | CO1 | (5) |
| 45. | Explain dot rule for coupled coils. | CO1 | (5) |
| 46. | Find the resonant frequency , Band width and Quality factor for the ideal series RLC circuit having L = 50 mH, C = 0.01 μF and Supply voltage V = 10 V. | CO2 | (5) |
| 47. | Calculate the total inductance of three coupled circuit shown in the figure. | CO1 | (5) |
| 48. | A three phase balanced delta connected load of 4+j8ῼ is connected across a 400V, 3-phase balanced supply. Determine the phase currents and line currents assume the phase sequence to be RYB. | CO1 | (5) |
| 49. | Derive the transient response of an RC circuit when it is switched to a unit step voltage source at time t=0. Assume that the initial voltage across capacitor is zero. | CO2 | (5) |
| 50. | For the given two port network find the impeadnce parameters. | CO2 | (5) |
| PART C( 2 X 10 = 20 MARKS) (ANSWER ANY TWO) |  |  |  |
| 51. | Find the current through 3Ω resistor, and the current supplied by the 7 V source by mesh analysis. | CO1 | (10) |
| 52. | With the help of phasor diagram, prove that with two wattmeter readings, real power, reactive power and powerfactor of a 3 circuit can be calculated. | CO2 | (10) |
| 53. | In the given circuit, the switch is closed at t=0. Derive the expression for transient current. | CO2 | (10) |

ALL THE BEST