Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14EE2001 | Duration : | 3hrs |
| Sub. Name : | Electric circuits & Networks | Max. marks : | 100 |

| Q. No. | Questions |  |  |  | Course outcome | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| PART-A (40X1=40 MULTIPLE CHOICE QUESTIONS) |  |  |  |  |  |  |
| 1. | Which of the circuit element given below is an active element?. |  |  |  |  |  |
|  | a. Voltage Source | b. Resistor | c.Inductor | d.Capacitor | CO1 | (1) |
| 2. | How much resistance is required to limit the current from a 12 V battery to 3.6 mA? |  |  |  |  |  |
|  | a. 33 KΩ | b. 3.3 KΩ | c. 2.2 KΩ | d. 22 KΩ | CO1 | (1) |
| 3. | 2500 in rectangular form is given by |  |  |  |  |  |
|  | a. 1.93-j0.52 | b. 1.93+j0.52 | c. 0.52+j1.93 | d. None of these. | CO2 | (1) |
| 4. | Ten lamps are connected in parallel across a 12 V supply. What is the voltage across each lamp |  |  |  |  |  |
|  | a. 120V | b. 1.2 V | c. 12V | d. 1200V | CO1 | (1) |
| 5. | Find RAB |  |  |  |  |  |
|  | a. 2Ω | b. . 3Ω | c. . 4Ω | d. . 6Ω | CO1 | (1) |
| 6. | When one of three series resistors is removed from a circuit and the circuit is reconnected, the current |  |  |  |  |  |
|  | a. Increases | b. decreases by the amount of current through the removed resistor | c. is zero | d. decreases by one-third | CO1 | (1) |
| 7. | Find I2 4I1 + 4I2 = 2 6I1 + 7I2 = 4 |  |  |  |  |  |
|  | a. 1A | b. -1A | c. 10 mA | d. 100 mA | CO1 | (1) |
| 8. | (2+j2) in polar form is given by: |  |  |  |  |  |
|  | a. 2450 | b. 2-450 | c. 2.828-450 | d. 2.828 450 | CO2 | (1) |
| 9. | . _________ law states that the algebraic sum of all the currents meeting at a junction is zero. |  |  |  |  |  |
|  | a. Faraday’s law | b. Kirchoff’s voltage law | c. Kirchoff’s current law | d. Ohms law | CO1 | (1) |
| 10. | Three equal resistance of 3 Ω are connected in star .What is the resistance in one of the arms in equivalent delta circuit? |  |  |  |  |  |
|  | a. 10Ω | b. . 3Ω | c. . 9Ω | d. . 27Ω | CO1 | (1) |
| 11. | Mesh Analysis is based on |  |  |  |  |  |
|  | a.KVL | b. KCL | c. None | d. Both KVL and KCL | CO1 | (1) |
| 12. | Find the voltage VAB if the current through the 2Ω resistor is 4 amps |  |  |  |  |  |
|  | a. 5 Volts | b. 6 Volts | c. 8 Volts | d. 10 Volts | CO1 | (1) |
| 13. | ___________ theorem is applicable only to a linear, bilateral net work |  |  |  |  |  |
|  | a. Superpostion | b. Thevinin’s | c. Norton’s | d. Reciprocity | CO1 | (1) |
| 14. | Superposition Theorem is not valid for |  |  |  |  |  |
|  | a. Power responses | b.voltage responses | c.current responses | d. all the three | CO1 | (1) |
| 15. | Calculate the Thevenin’s Resistance across terminals a and b of the circuit in Figure. |  |  |  |  |  |
|  | a. 4ῼ | b.5ῼ | c.3ῼ | d.6ῼ | CO1 | (1) |
| 16. | The equivalent inductance of the circuit shown below, Leq = _______. |  |  |  |  |  |
|  | a. 9H | b. 1H | c. 3H | d. 15H | CO1 | (1) |
| 17. | In the circuit shown below, determine the value of RL at which maximum power will be transferred to the load |  |  |  |  |  |
|  | a. 1Ω | b. . 5Ω | c. . 10Ω | d. . 20Ω | CO1 | (1) |
| 18. | What theorem replaces a complex network with an equivalent circuit containing a source voltage and a series resistance? |  |  |  |  |  |
|  | a. Norton | b. Multinetwork | c.Thevenins | d. Superposition | CO1 | (1) |
| 19. | Norton’s theorem is the dual of------------ |  |  |  |  |  |
|  | a. Superposition | b. Thevenins | c. Millmans | d. Reciprocity | CO1 | (1) |
| 20. | Under the conditions of maximum power transfer, the efficiency is ______ |  |  |  |  |  |
|  | a. 50% | b.100% | c.75% | d.90% | CO1 | (1) |

| 21. | When XC = XL the circuit: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | a. Draws maximum current | b. is at resonance | c. applied voltage is zero | d. draws minimum current | CO1 | (1) |
| 22. | If the value of C in a series RLC circuit is decreased, the resonant frequency |  |  |  |  |  |
|  | a. is reduced to zero | b. is not affected | c. increases | d. decreases | CO2 | (1) |
| 23. | A resonant circuit has a lower critical frequency of 7 kHz and an upper critical frequency of 13 kHz. The bandwidth of the circuit is |  |  |  |  |  |
|  | a. 10KHz | b. 12KHz | c. 6KHz | d. 8KHz | CO2 | (1) |
| 24. | In a Y-connected circuit, between each line voltage and the nearest phase voltage, there is a phase angle of |  |  |  |  |  |
|  | a. 30° | b. 0° | c. 90° | d. 60° | CO1 | (1) |
| 25. | What is the maximum value of co-efficient of coupling? |  |  |  |  |  |
|  | a. 1.2 | b. 0.5 | c. 0.8 | d. 1.0 | CO1 | (1) |
| 26. | In a star-connected system, the relation between line voltage VL and Phase voltage Vph is |  |  |  |  |  |
|  | a. | b. | c. | d. None of these. | CO1 | (1) |
| 27. | Two inductively coupled coils have self inductance L1 = 0.02 H and L2 = 0.01H. If the Coefficient of coupling K = 0.5, The value of mutual inductance between the coil is |  |  |  |  |  |
|  | a. 0.00707 H | b. 0.00707 mH | c. 7 H | d. 700 H | CO1 | (1) |
| 28. | In a three-phase system, when the loads are perfectly balanced, the neutral current is |  |  |  |  |  |
|  | a. two-thirds of maximum | b. zero | c. one-third of maximum | d. at maximum | CO1 | (1) |
| 29. | In two wattmeter method of power measurement if the total power is measured by one wattmeter only then pf of the system is |  |  |  |  |  |
|  | a. 0 | b. 0.5 | c. 1 | d. none of the above | CO1 | (1) |
| 30. | In a three-phase system, the voltages are separated by |  |  |  |  |  |
|  | a. 120° | b. 180° | c. 45° | d. 90° | CO1 | (1) |
| 31. | In a certain Y-Y system, the source phase currents each have a magnitude of 10 A. The magnitude of each load current for a balanced load condition is |  |  |  |  |  |
|  | a. 3A | b. 12A | c. 10A | d. 27A | CO1 | (1) |
| 32. | The time constant of RC circuit with R=200 ohms and C=10 microfarad is ------------ |  |  |  |  |  |
|  | a. 0.2 | b. 0.002 | c. 2 | d. 0.4 | CO2 | (1) |
| 33. | Resonant frequency of a series RLC circuit is given by |  |  |  |  |  |
|  | a. fr=2πLC | b. fr=12πLC | c. fr=LC2π | d. fr=2πLC | CO2 | (1) |
| 34. | The time constant of a series RL Circuit is __________ |  |  |  |  |  |
|  | a. τ = | b. τ = R2L | c. τ = RL | d. τ = | CO2 | (1) |
| 35. | Which one of following graphs represents the nature of variation of the potential drop V across the resistance with time in series RL circuit? |  |  |  |  |  |
|  | a. | b. | c. | d. | CO2 | (1) |
| 36. | Which among the following is regarded as open circuit input impedance? |  |  |  |  |  |
|  | a. Z11 | b. Z12 | c. Z21 | d. Z22 | CO1 | (1) |
| 37. | If the resistance in parallel with a parallel resonant circuit is reduced, the bandwidth |  |  |  |  |  |
|  | a. Increases | b. becomes sharper | c. disappears | d. decreases | CO2 | (1) |
| 38. | How is the short circuit reverse transfer admittance (y12) calculated in terms of current and voltage ratio? |  |  |  |  |  |
|  | a. V2/ I1 (keeping I2 = 0) | b. I2/ V1 (keeping V2 = 0) | c. I1/ V2 (keeping V1 = 0) | d. V1/ I2 (keeping I1 = 0) | CO1 | (1) |
| 39. | A 2-port network using z-parameter representation is said to be reciprocal if |  |  |  |  |  |
|  | a. Z11 = Z22 | b. Z12 = Z21 | c. Z12 = - Z21 | d. Z11 Z22 = - Z12Z21 = 1 | CO1 | (1) |
| 40. | A ________ filter significantly attenuates all frequencies below fc and passes all frequencies above fc. |  |  |  |  |  |
|  | a. low-pass | b. high-pass | c. band-pass | d. band-stop | CO3 | (1) |

| PART B(8 X 5 = 40 MARKS) (ANSWER ANY EIGHT) |  |  |  |
| --- | --- | --- | --- |
| 41. | Find the unknown currents using KCL. | CO1 | (5) |
| 42. | How much voltage is dropped across R3 in the given circuit? | CO1 | (5) |
| 43. | Determine the current through R3 by node analysis. | CO1 | (5) |
| 44. | State superposition theorem. Mention its applications. | CO1 | (5) |
| 45. | Find the value of Rout such that maximum power is delivered to it. Also, find the maximum power. | CO1 | (5) |
| 46. | A series-connected circuit has R = 4 Ω and L = 25 mH. (a) Calculate the value of C that will produce a quality factor of 50. (b) Find lower and higher cut-off frequencies. c) bandwidth | CO2 | (5) |
| 47. | Two coupled coils with L1=0.5 mH and L2=0.4mH have a coupling coefficient k=0.75. Find the mutual inductance and the turns ratio. If current in the first coil is 10sin317t find the mutually induced voltage in the second coil. | CO2 | (5) |
| 48. | A balanced star connected load of (8+j6)Ω/phase is supplied with a balanced 3-, 400V supply. Find the a) phase current b)line current c) Real power d) reactive power e) apprant power. | CO2 | (5) |
| 49. | A series RLC circuit with R=5Ω, L=0.1 H, C=100µF is applied with a step voltage of 150 volts. Find the nature of damping and write the expression for transient current. | CO2 | (5) |
| 50. | Design a constant K low pass filter having a cut off frequency of 2 KHz to operate with a terminated load resistance of 500 ohm. | CO3 | (5) |
| PART C( 2 X 10 = 20 MARKS) (ANSWER ANY TWO) |  |  |  |
| 51. | Find the load current using mesh analysis. | CO1 | (10) |
| 52. | Derive an expression to find the output current, voltage and amplification factor of a single tuned coupled circuit | CO2 | (10) |
| 53. | Determine the open circuit impedance parameters of the π network given below. | CO2 | (10) |

ALL THE BEST