Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14EE2033 | Duration : | 3hrs |
| Sub. Name : | HARMONICS & POWER QUALITY | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a. |  | Classify and illustrate the typical power quality phenomena defined in IEEE 1159. | CO2/CO1 | 10 |
|  | b. |  | Draw the CBEMA & ITI curve and explain about events described in the curves. | CO2 | 10 |
| (OR) |  |  |  |  |  |
| 2. | a. |  | Examine the source and effects of different categories of short duration voltage Variations. | CO2 | 10 |
|  | b. |  | Give outlines on IEEE and IEC power quality standards. | CO3 | 10 |
| 3. | a. |  | In a three-phase system, the load voltage consists of Va=0.9sinωt, Vb=0.8sin(ωt-120), Vc=0.7sin(ωt+120). Calculate the following quantity with a sag period of 10ms, 20ms, 30ms with respect to ABC phases. a) Detroit Edison Sag Score (SS) b) Voltage sag Energy c) Voltage sag lost energy index (VSLEI) | CO2/ CO1 | 8 |
|  | b. |  | Explain the different types of sag with a relevant vector diagram with proper equations. | CO2/CO3 | 12 |
| (OR) |  |  |  |  |  |
| 4. |  | Explain the following causes of sags. - Voltage sag due to motor sag - Voltage sag due to single line to ground fault. Voltage sag due to transformer energizing. |  | CO2/CO3 | 20 |
| 5. |  | Give outlines on sources of over voltage due to the following phenomenon a) Capacitor switching b) Lightning |  | CO2/CO3 | 20 |
| (OR) |  |  |  |  |  |
| 6. |  | What is the need for protection against over voltages? What are the basic principles of over voltages protection of load equipments? |  | CO2/CO1 | 20 |
| 7. | a. |  | The converter circuit is shown the below fig. Calculate the following quantities (a)THD of AC mains current (b) DIN (c) TDD of AC mains current (d) DF (e)DPF (f) PF | CO2/ CO3 | 12 |
|  | b. |  | Discriminate between linear and non linear load | CO2 | 8 |
| (OR) |  |  |  |  |  |
| 8. |  | Explain in detail about general procedure for harmonic distortion evaluation at the point of coupling, utility systems, customer facility and industrial facility. |  | CO2/ CO3 | 20 |
|  |  |  | Compulsory: |  |  |
| 9. | a. |  | Illustrate the principle of DVR operation used for sag mitigation | CO2/CO1 | 10 |
|  | b. |  | Discuss the construction and working principle of active filters for harmonic mitigation. | CO2/CO1 | 10 |

ALL THE BEST