Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14FP2010 | Duration : | 3hrs |
| Sub. Name : | Unit Operations in Food Process Engineering-II | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain the principle and theory of diffusion process | CO1 | 10 |
|  | b. | Explain the flash distillation process with a neat diagram. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the batch distillation process with a neat sketch. | CO2 | 10 |
|  | b. | Calculate the vapour and liquid compositions in equilibrium at 95℃ for benzene-touene using the vapour pressure at 101.32 kPa. The partial pressures of benzene and toluene are 155.7 kPa and 63.3 kPa respectively. | CO2 | 10 |
| 3. | a. | Discuss the principles of extraction and briefly explain the applications of extraction in food processing. | CO1 | 10 |
|  | b. | Explain the functioning of single stage batch extraction process with diagram. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Explain in detailabout the Super Critical Fluid Extraction process with a neat diagram and list out the applications of Super Critical Fluid Extraction in food processing. | CO2 | 20 |
| 5. | a. | Explain forced circulation liquid evaporator crystallizer with a diagram. | CO2 | 10 |
|  | b. | What is the size of a nucleus of a sugar crystal in equilibrium at 20℃ with mother liquor at 115% supersaturation? The surface tension at the crystal-liquid interphase is estimated to be 0.3 ergs/cm2. The density of the sugar crystal is 1600 kg/m3 and molecular weight of sucrose(M) is 342. Consider the value of V as 1. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Write a note on the principle of crystallization. | CO1 | 5 |
|  | b. | Describe a batch crystallizer with a neat diagram and explain its application in food processing. | CO2 | 10 |
|  | c. | Discuss the importance of mixing and agitation in crystallization process. | CO2 | 5 |
| 7. | a. | Explain the solvent transport phenomena in ultra filtration process. | CO2 | 10 |
|  | b. | The concentration of whey is being accomplished by using an ultrafiltration membrane to separate water. The 400 kg/hour of feed stream has 7 % solids and is being increased to 22 % total solids. The membrane tube has a 6 cm inside diameter and the pressure difference is 2100 kPa. Estimate the flux of water through the membrane and length of the membrane tube when the hydraulic permeability is 3.7x10-5 kg of water/m2k Pa. s | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | List out the desirable characteristics of membrane configurations? Describe briefly the spiral wound and tubular membrane configurations. | CO1 | 10 |
|  | b. | An ultrafiltration membrane was examined microscopically and found to have about 1,50,000 pores with an average diameter of 0.8 x 10-6 m per mm2 of membrane surface. The thickness of the membrane is 160x10-6 m. The viscosity of the permeate is 0.0013 Pa.s. Estimate the following : a) Porosity of the membrane (€), b) Hydraulic Permeability (LP) and c) Permeate Flux(J) for a Trans membrane Pressure Difference(∆PTM) of 1.5 Pa. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the process of adsorption in columns. | CO1 | 10 |
|  | b. | Discuss the working of bubble columns for gas absorption process with neat sketch. | CO2 | 10 |

ALL THE BEST