Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14ME2016 | Duration : | 3hrs |
| Sub. Name : | THERMAL ENGINEERING II | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain with neat sketch the port timing diagram of 2 stroke petrol engine. | CO1 | 10 |
|  | b. | Explain the working principle of battery ignition system with a neat diagram. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. |  | An engine develops 20 kW of brake power with a mechanical efficiency of 80% consuming petrol at the rate of 5.8 kg/hr. Calculate (a) Brake thermal efficiency (b) Specific fuel consumption and (c) Indicated thermal efficiency if the calorific value of the fuel is 42,000 kJ/kg. | CO1 | 20 |
| 3. |  | Derive an expression for air standard efficiency of dual cycle. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Air enters in to a constant volume cycle at 27° C and 1 bar and the compression ratio is 7.5.The maximum Temperature of the cycle is 1000 K. Find the Air Standard efficiency and Mean effective pressure of the cycle. | COI | 20 |
| 5. |  | An air-conditioning system is to be designed for a restaurant with the following data: Outside design conditions : 400C DBT and 280C WBT Inside design conditions : 250C DBT and 50% RH Solar heat gain through walls, roof and floor : 5.87 kW Solar heat gain through glass : 5.52 kW Occupants : 25 Sensible heat gain per person : 58 kW Latent heat gain per person : 58 kW Lightening load : 2.3 kW Sensible heat gain from other sources : 11.63 kW Infiltrated air : 15 m3/min Determine Room sensible heat factor. | CO4 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Explain summer air-conditioning system for hot and dry weather with a neat sketch. | CO4 | 20 |
| 7. |  | A conical diffuser has entry and exit diameters of 15 cm and 30 cm respectively. The pressure, temperature and velocity of air at entry are 0.89 bar, 340 K and 180 m/s. Determine the exit pressure, exit velocity and the force exerted on the diffuser walls. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Air at p1= 3bar, T1= 288K and M1=1.5 is brought to sonic velocity in a frictionless constant area duct through which heat transfer can occur. Determine the final pressure and temperature, and heat added during the process. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain open cycle constant pressure gas turbine with a neat sketch. | CO3 | 10 |
|  | b. | Explain closed cycle constant pressure gas turbine with a neat sketch. | CO3 | 10 |