Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 14ME3035 | Duration : | 3hrs |
| Sub. Name : | SOLAR THERMAL ENERGY CONVERSION | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Calculate the transmittance, reflectance and absorptance of a single glass cover 2.4 mm thick at an angle of 62⁰. The extinction coefficient of the glass is 34 m-1 and the refractive index of the glass is 1.527. | CO1 | 14 |
|  | b. | Discuss the working of the instrument used for measuring the Sunshine Duration. | CO1 | 6 |
| (OR) |  |  |  |  |
| 2. | a. | Find the Solar declination, Solar time and Equation of Time (ET) for Coimbatore on September 8. The latitude and longitude values of Coimbatore are 11.02° N, 76.97° E. Take local time as 11:00AM. | CO1 | 8 |
|  | b. | With neat diagram, explain the construction and working principle of Pyranometer. | CO1 | 8 |
|  | c. | Discuss the need for selective surface coating on solar collectors? | CO1 | 4 |
| 3. | a. | With neat diagram, discuss the following terms: - Terrestrial Solar Radiation - Extra-terrestrial Solar Radiation - Beam Radiation - Diffuse Radiation | CO1 | 12 |
|  | b. | Illustrate the radiative properties and characteristics of materials with help of first law of thermodynamics. | CO1 | 8 |
| (OR) |  |  |  |  |
| 4. | a. | Calculate the top loss coefficient for an absorber with a single glass cover having the following specifications: Plate-to-cover spacing = 17mm Plate emittance = 0.94 Ambient air and sky temperature = 13°C Wind heat transfer coefficient = 13W/m2°C Mean plate temperature = 104°C Collector tilt = 44° Glass emittance = 0.87 | CO2 | 10 |
|  | b. | Define the term area concentration ratio and with help of schematic diagram and obtain the maximum concentration ratio of circular concentrators. | CO2 | 10 |
| 5. | a. | Briefly discuss the role of Phase Change Materials in thermal energy storage system. | CO3 | 10 |
|  | b. | In thermal energy storage system, explain the concept chemical energy storage and list down the applications. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | In a home, there are seven persons, the hot water demand is 110litres /person and the average solar radiation in that latitude is 560W/m2. The ambient temperature is 29°C and the inlet water temperature is 2°C less than the ambient temperature. Required outlet temperature of water it should be 53°C. Calculate the area of solar collector to meet the required demand and cost of the solar collector area if the per square meter cost is Rs.578/-. | CO4 | 15 |
|  | b. | List down the primary types of Solar Concentrating collectors. | CO2 | 5 |
| 7. | a. | With neat diagram, explain the direct circulation water heating system with inclusion of storage tank. | CO4 | 10 |
|  | b. | Discuss the concept of solar deslination and explain the techniques involved to achieve desalination. | CO4 | 10 |
| (OR) |  |  |  |  |
| 8. |  | With help of neat block diagram, explain the absorption air conditioning system in solar refrigeration. | CO4 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | With help of neat diagram, explain the Solar Electric Generating System under parallel operation with the source of solar and natural gas. | CO4 | 15 |
|  | b. | Discuss the practical difficulties for usage of solar furnace in solar thermal applications. | CO4 | 5 |

ALL THE BEST