Reg.No. ____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 16EC2004 | Duration : | 3hrs |
| Sub. Name : | ELECTRON DEVICES AND CIRCUITS | Max. marks : | 100 |

| Q. No. | Questions |  |  |  | Course outcome | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| PART-A (40X1=40 MULTIPLE CHOICE QUESTIONS) |  |  |  |  |  |  |
| 1. | The ratio of which two currents is represented by β? |  |  |  | CO1 |  |
|  | a. IC and IB | b. IC and IE | c. IE and IB | d. None of the above |  | (1) |
| 2. | At what region of operation is the base-emitter junction forward biased and the base-collector junction reverse biased? |  |  |  | CO1 |  |
|  | a.Saturation | b. active | c. Cutoff | d. None of the above |  | (1) |
| 3. | For the BJT to operate in the saturation region, the base-emitter junction must be _____-biased and the base-collector junction must be _____-biased. |  |  |  | CO1 |  |
|  | a. forward, forward | b. forward, reverse | c. reverse, reverse | d. reverse, forward |  | (1) |
| 4. | Transistors are _____terminal devices. |  |  |  | CO1 |  |
|  | a.1 | b.2 | c.3 | d.4 |  | (1) |
| 5. | If too large current passes through the diode |  |  |  | CO1 |  |
|  | a.all electrons will leave | b. Excessive heat will damage the diode | c. All holes will freeze | d.diode will emit light |  | (1) |
| 6. | For a properly biased pnp transistor, let IC = 10 mA and IE = 10.2 mA. What is the level of IB? |  |  |  | CO1 |  |
|  | a. 200µA | b.200mA | c.0.2A | d.20A |  | (1) |
| 7. | How much is the base-to-emitter voltage of a transistor in the "on" state? |  |  |  | CO2 |  |
|  | a. Undefined | b. 0.7V | c.0.7mV | d.7V |  | (1) |
| 8. | Use this table of collector characteristics to calculate β at VCE = 15 V and IB = 30 µA |  |  |  | CO1 |  |
|  | a.50 | b.106 | c. 100 | d.400 |  | (1) |
| 9. | Tuned amplifiers are commonly used whenever ________________is required. |  |  |  | CO2 |  |
|  | a. High focus | b.high bandgap | c. High selectivity | d.low light emission |  | (1) |
| 10. | In a common-emitter configuration _____ is the controlling current while _____ is the controlled current. |  |  |  | CO1 |  |
|  | a. IB, IC | b. IC, IE | c. IC, IB | d. None of the above |  | (1) |
| 11. | The circuit that provides the best stabilization of operating point is |  |  |  | CO2 |  |
|  | a.Base resistor bias | b.collector feedback bias | c. voltage divider bias | d. None of the above |  | (1) |
| 12. | The disadvantage of base resistor method of transistor biasing is that it |  |  |  | CO2 |  |
|  | a.Is complicated | b.Is sensitive to changes in ß | c. Provides high stability | d. None of the above |  | (1) |
| 13. | JFET is a __________ controlled device. |  |  |  | CO2 |  |
|  | a.current | b. power | c. Frequency | d. voltage |  | (1) |
| 14. | FET has high input resistance than BJT |  |  |  | CO2 |  |
|  | a. True | b.False | c.Partially True | d.None of the above |  | (1) |
| 15. | Position of Q point for a class ______ amplifier is in the middle of the active region |  |  |  | CO1 |  |
|  | a. B | b. C | c. AB | d. A |  | (1) |
| 16. | In class ------------ amplifier the transistor remains active for less than half cycle. |  |  |  | CO2 |  |
|  | a. B | b. C | c. AB | d. A |  | (1) |
| 17. | For a differential amplifier the input impedance is ___________ |  |  |  | CO2 |  |
|  | a.high | b.low | c.zero | d.infinity |  | (1) |
| 18. | A majority of the ----------------------- that is drawn from the supply by a class-A amplifier is used up by the amplifier itself |  |  |  | CO2 |  |
|  | a. current | b. power | c. efficiency | d. voltage |  | (1) |
| 19. | The equation of drain current for Enhancement type MOSFET |  |  |  | CO1 |  |
|  | a. ID= K(VGS- VGS(th))² | b. ID= K(VGS(th)-VGS)² | c. ID= K(VGS -VGS(th)) | d. ID= (VGS-VGS(th))² |  | (1) |
| 20. | Determine the value of α when β = 100. |  |  |  | CO1 |  |
|  | a. 1.01 | b. 101 | c.0.99 | d. Cannot be solved with the information provided |  | (1) |

| 21. | The crystal oscillator works on the principle of __________ |  |  |  | CO1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | a. Lenz law | b. electromagnetic induction | c. piezoelectric | d. inverse piezoelectric |  | (1) |
| 22. | An oscillator always needs an amplifier with |  |  |  | CO2 |  |
|  | a. Positive feedback | b. Negative feedback | c. Both types of feedback | d. no feedback |  | (1) |
| 23. | The value of loop gain factor which is equal to 1 produces ______________ oscillations |  |  |  | CO3 |  |
|  | a. growing | b. no | c. decaying | d. sustained |  | (1) |
| 24. | In a RC phase shift oscillator, each RC circuit produces _______ degree phase. |  |  |  | CO2 |  |
|  | a. 90 | b. 45 | c. 60 | d. 180 |  | (1) |
| 25. | Sweep oscillators produces _________ waves |  |  |  | CO1 |  |
|  | a. saw tooth | b. triangular | c. square | d. rectangular |  | (1) |
| 26. | If the open loop gain is 20 and the beta value is 0.05. Find the closed loop gain |  |  |  | CO2 |  |
|  | a. 0 | b. 20 | c. infinity | d. 2 |  | (1) |
| 27. | Find the resonant frequency of RC phase shift oscillator if R = 8K ohm and C = 3 nano farad |  |  |  | CO2 |  |
|  | a. 3.7 KHz | b. 1 KHz | c. 2.7 KHz | d. 0.7 KHz |  | (1) |
| 28. | In a crystal oscillator at higher frequencies XL is high and XC is low, so the crystal is seen by the circuit as |  |  |  | CO1 |  |
|  | a. no change | b. resistive | c. capacitive | d. inductive. |  | (1) |
| 29. | The common Noise of input signal in differential amplifier |  |  |  | CO2 |  |
|  | a. not affected | b. increases | c. decreases | d. remains the same |  | (1) |
| 30. | The common-emitter cascade amplifier provides |  |  |  | CO2 |  |
|  | a. high voltage gain | b. high current gain | c. high power gains | d. all of above |  | (1) |
| 31. | Class _____ amplifiers are normally operated in a push-pull configuration in order to produce an output that is a replica of the input. |  |  |  | CO1 |  |
|  | a. B | b. C | c. AB | d. D |  | (1) |
| 32. | Class A operation can achieve power efficiency of over _____. |  |  |  | CO1 |  |
|  | a. 50% | b. ) 25% | c. 78.5% | d. 90% |  | (1) |
| 33. | MOSFET can be operated with |  |  |  | CO2 |  |
|  | b. Positive voltage | c. Negative voltage | c.NO voltage | d.none of the above |  | (1) |
| 34. | MOSFET is sometimes called as ________ FET |  |  |  | CO1 |  |
|  | a.Insulated gate | b.isolated gate | c.powered gate | d.depleted gate |  | (1) |
| 35. | Impedance of tuned circuit |  |  |  | CO3 |  |
|  | a. Zr = L / CR | b. Zr = X / CR | c. Zr = L / C | d. Zr = L / R |  | (1) |
| 36. | For a DC analysis of an amplifier |  |  |  | CO2 |  |
|  | a. Coupling capacitor is open circuited | b. bypass capacitor is open circuited | c. AC source shorted | d.all the above |  | (1) |
| 37. | For an AC analysis of an amplifier |  |  |  | CO2 |  |
|  | a. Coupling capacitor is open circuited | b. Coupling capacitor is short circuited | c. Coupling capacitor value is increased | d. Coupling capacitor value is decreased |  | (1) |
| 38. | In a JFET voltage divider biasing circuit R1 and R2 are used to |  |  |  | CO1 |  |
|  | a. keep the gate-source junction in reverse bias. | b. keep the gate-source junction in forward bias. | c. . keep the gate-source junction in no bias. | d.None of the above |  | (1) |
| 39. | Self Bias is a JFET Biasing circuit that uses a ___________to help reverse bias the JFET gate. |  |  |  | CO2 |  |
|  | a. source resistor | b.Drain resistor | c.Gate resistor | d.None of the above |  | (1) |
| 40. | Important feature of an RC Oscillator is its _________________________ |  |  |  | CO2 |  |
|  | a. frequency stability | b.Power | c.efficiency | d.voltage gain |  | (1) |

| PART B(8 X 5 = 40 MARKS) (ANSWER ANY EIGHT) |  |  |  |
| --- | --- | --- | --- |
| 41. | Using the following data: IE = 10 mA, and hFE = 100 to 300.Prove that the The Q-point of voltage divider bias circuit of a BJT is less dependent on hFE | CO3 | (5) |
| 42. | Explain the working of Depletion type MOSFET with necessary equations and graph. | CO2 | (5) |
| 43. | Differentiate an ideal diode from a practical diode. | CO2 | (5) |
| 44. | Determine the q-point values of IC and VCE at both of these temperatures. Given VBE = 0.7 V | CO3 | (5) |
| 45. | A single tuned amplifier consist of tuned circuits having R=5Ω, L=10mh,C=0.1µf. Determine a) resonant frequency b) quality factor of tank circuit c) band width of amplifier. | CO3 | (5) |
| 46. | Differentiate between class A and class B amplifier. | CO2 | (5) |
| 47. | Draw the Electrical Equivalent Circuit Diagram of a Quartz Crystal and state the what does R,L and C represent? |  | (5) |
| 48. | Draw the block diagram of an oscillator and state the barkhausen criterion. | CO2 | (5) |
| 49. | Prove that G=G1 x G2 x G3 considering the input to first stage as V and the output of first stage = G1V. | CO3 | (5) |
| 50. | Derive equations for JFET voltage divider bias. | CO2 | (5) |
| PART C( 2 X 10 = 20 MARKS) (ANSWER ANY TWO) |  |  |  |
| 51. | Illustrate the voltage divider biasing technique in a BJT. | CO2 | (10) |
| 52. | Detail out the working of an RC phase shift oscillator with related circuit diagrams and supporting expressions. | CO3 | (10) |
| 53. | Perform an AC analysis of the circuit shown below. | CO3 | (10) |

ALL THE BEST