UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 09AE207 / 12 AE220 | Duration : | 3 hrs |
| Sub. Name : | Gas Dynamics | Max. marks : | 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |
| 1. | Define Mach angle. | (1) |
| 2. | Differentiate subsonic and supersonic flows with respect to the speed of sound. | (1) |
| 3. | When is a shock called as a weak shock ? | (1) |
| 4. | A supersonic expansion or compression with Mach lines which are straight is known as _______________ region. | (1) |
| 5. | The small perturbation equations for subsonic and supersonic flows are _______________ . | (1) |
| 6. | Define critical Mach number. | (1) |
| 7. | Mixed subsonic and supersonic flow fields are known as _____________________ . | (1) |
| 8. | The conditions for divergence of railing edge pressure correspond to a rapid drop in lift coefficient and to the onset of certain unsteady flow phenomenon known as _______________ . | (1) |
| 9. | The high speed wind tunnels in which the Mach number ranges from 1.2 to 5 are known as _____________ . | (1) |
| 10. | ___________________________ are electromechanical devices that convert pressure to electrical signals which can be recorded using a data system. | (1) |

| PART B(5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11 | Draw the flow fields generated in a convergent-divergent nozzle for various back pressures downstream of the nozzle exit. | (3) |
| 12 | What are shock tubes ? Name few applications of shock tubes. | (3) |
| 13 | State Crocco’s theorem for two dimensional flows. | (3) |
| 14 | State the components of aircraft drag at supersonic speeds. | (3) |
| 15 | Name the three optical flow visualization techniques commonly used for visualizing compressible flows. | (3) |

| PART C(5 X 15= 75 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 16. | a. | A ramjet flies at 11 km altitude with a flight Mach number of 0.9. In the inlet diffuser, the air is brought to the stagnation condition so that it is stationary just before the combustion chamber. Combustion takes place at constant pressure and a temperature increase of 1500K results. The combustion products are then ejected through the nozzle. Calculate (a) stagnation pressure and temperature (b) exit velocity at the nozzle. Take at inlet P=0.3 atm and T=213 K and at exit Pexit = 0.3 atm. | (15) |
| (OR) |  |  |  |
| 17. | a. | A De Laval nozzle has to be designed for an exit Mach number of 1.5 with an exit diameter of 200 mm. Find the required ratio of throat area to exit area. The reservoir conditions are given as P0 = 1 atmg ; T0=20C. Find also the maximum mass flow rate through the nozzle. What will be the exit pressure and temperature? |  |
| 18. | a. | A shock of strength 0.382 is generated by a piston which moves in sill air at 300 K. Determine the shock speed and the piston speed. | (15) |
| (OR) |  |  |  |
| 19. | a. | Design a nozzle to deliver Mach 1.6 air flow with inlet diameter 40 mm, semi-convergence angle 30 and semi-divergence angle 7, to be run by a reservoir at 6 atm and 35C, with a mass flow rate of 0.11 kg/s, discharging to atmospheric pressure. | (15) |
| 20. | a. | A given profile has at M=0.29 the following lift coefficients : CL = 0.2 at =3, CL = -0.1 at = -2 where  is the angle of attack. Plot the relation showing dCL/d vs M for the profile values of M upto 1.0. | (10) |
|  | b. | Explain the application of Prandtl-Glauert rule to wings of finite span. | (5) |
| (OR) |  |  |  |
| 21. | a. | Derive the general potential equation for three dimensional compressible flows. | (10) |
|  | b. | If the value of the pressure coefficient at the maximum velocity location on a profile is -2.13, determine the critical Mach number. | (5) |
| 22. | a. | Explain the phenomenon of transonic flow past swept wings and compare the transonic drag polar of an unswept and swept wing using CL vc CD plot. | (15) |
| (OR) |  |  |  |
| 23. | a. | Estimate the skin friction coefficient for a turbulent boundary layer on a flat plate, when Me = 2.5, Rex = 6.142 x 106 and Tw = 3.0Te. Take Fc = 2.056 and FRe = 0.1729. | (15) |
| 24. | a. | Describe the construction and working of a Schlieren system employed for flow visualization studies with a neat sketch. | (15) |
| (OR) |  |  |  |
| 25. | a. | Explain the construction and working of the wind tunnels with neat sketches. - Blow down type wind tunnels - Induction type wind tunnels | (15) |

ALL THE BEST