UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 09BT203 | Duration : | 3 hrs |
| Sub. Name : | Principles of Chemical Engineering | Max. marks : | 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |
| 1. | Give the unit for velocity_________ | (1) |
| 2. | Give the unit for volumetric flow rate________ | (1) |
| 3. | Express the ideal gas law equation | (1) |
| 4. | State the law of conservation of mass | (1) |
| 5. | Rotameter is used to measure___________ | (1) |
| 6. | What is the specific gravity of water_________ | (1) |
| 7. | ________is a form of energy that flows from one body to another with tempertature difference | (1) |
| 8. | Pressure drop is expressed as_______ | (1) |
| 9. | What is crystallization? | (1) |
| 10. | Define molarity | (1) |

| PART B(5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11 | Differentiate Venturi meter and orifice meter | (3) |
| 12 | State in brief the principle of a centrifugal pump | (3) |
| 13 | Differentiate velocity and volumetric flowrate | (3) |
| 14 | What is coefficient of discharge? How is it expressed | (3) |
| 15 | Give the principle of a positive displacement pump | (3) |

| PART C(5 X 15= 75 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 16. | a. | A gaseous mixture has the following composition by volume: CO2 - 8%, CO - 14%, O2 - 6%, H2O - 5%, CH4 - 1% and N2 - 66%. Calculate: i. Average molecular weight of gas mixture ii. Density of gas mixture at 303 K and 101.325 kPa. | (10) |
|  | b. | A cylinder contains 15 kg of liquid propane. What volume in m3 will propane occupy if it is released and brought to NTP conditions? | (5) |
| (OR) |  |  |  |
| 17. | a. | An aqueous solution contains 15% ethanol by volume. Find the weight % of ethanol if densities of ethanol and water are 0.79 g/cm3 and 1.0 g/cm3 respectively. | (5) |
|  | b. | A certain quantity of gas contained in a closed vessel of volume 1 m3 at temperature of 298 K and pressure of 121.59 kPa is to be heated such that pressure should not exceed 405 kPa. Calculate temperature of gas attained. | (5) |
|  | c. | Assuming air to contain 79% N2 and 21% O2 by volume. Calculate the density of air at NTP condition. | (5) |
| 18. | a. | The dilute acid containing 25% H2SO4 is concentrated by commercial grade H2SO4 containing 98% H2SO4 to obtain desired acid containing 65% H2SO4. Find the quantities of the acids required to make 1000 kg of desired acids. | (8) |
|  | b. | In manufacture of acetic acid by oxidation of acetaldehyde, 100 kmol of acetaldehyde is fed to a reactor per hour. The product leaving the reactor contains 14.81% acetaldehyde, 59.26% acetic acid and rest is oxygen on mole basis. Find the percentage conversion of acetaldehyde. | (7) |
| (OR) |  |  |  |
| 19. | a. | Explain on Evaporation, Drying and Crystallization with diagrams whenever necessary | (15) |
| 20. | a. | Using Buckingham’s π-theorem method, show that the discharge Q consumed by an oil ring depends upon the internal diameter of the ring D, rotational speed N, density ρ, iscosity µ, surface tension σ and the specific weight of oil w. | (10) |
|  | b. | Discuss the principle and operation of different types of simple manometers with a neat diagram. | (5) |
| (OR) |  |  |  |
| 21. | a. | Derive the Bernoulli’s equation and state the assumptions made for such a derivation. | (15) |
| 22. | a. | Explain the principle, operation and applications of Pitot tubes with a neat diagram. | (7) |
|  | b. | A horizontal venturimeter with inlet and throat diameter 30 cm and 15 cm respectively is used to measure the flow of water. The reading of differential manometer connected to the inlet and the throat is 20 cm of mercury. Determine the rate of flow. Take Cd = 0.98. | (8) |
| (OR) |  |  |  |
| 23. | a. | Discuss the mechanism of fluidization. | (5) |
|  | b. | Explain the various types of notches and weir types with neat diagrams. | (10) |
| 24. | a. | Explain the principle, operation and applications of positive displacement pump with a neat diagram | (15) |
| (OR) |  |  |  |
| 25. | a. | Give the detailed operation of the venture meter and orifice meter with diagrams whenever necessary | (15) |

ALL THE BEST