UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.  No. ________

End Semester Examination – Nov / Dec – 2016

Subject Title: UNIT OPERATIONS			                            	Time: 3 hour

Subject Code: 09BT205 								Maximum Marks: 100

Answer ALL questions

PART- A ( 10 x 1 =10 marks)

1.	State the Basic laws of heat conduction

2.	What is the driving force   in heat transfer?

3.	Convection classified into -------and ---------.

4.	Write the thermal conductivity unit.

5.	What is LMTD?

6.	List out the evaporator feeding methods.

7.	What is the function of filter aids? Give examples.

8.	The drum for the vacuum filters turns with a speed of--------

9.	Write down the significance of baffles.

10.	Define unit operations and few examples.

PART- B( 5 x 3 =15 marks  )

11.	State and explain the Fourier’s law of heat conduction.

12.	Write the main difference between natural and forced convection.

13.	Explain steam economy and evaporator capacity.

14.	Define free settling and hindered settling.

15	What is the need for mixing and agitation?

PART- C (5 x 15 =75 marks )

16.	a.	A steam pipe line 6.5 cm outside diameter is lagged with a 5cm layers of 	asbestos (thermal 	=0.12) and  a 4 cm  layers of cork( thermal conductivity= 	0.03) .If the temperature of the 	outer 	surface of the pipe is 150 ºC and 	temperature of the outer surface of the cork  is 	35 ºC. 	Calculate heat loss in 	calories per hour per meter.	            	          	          (8)

b.	Derive the expression for heat transfer through furnace wall made of three 	different materials in series. Assume k1, k2 and k3   be the thermal conductivities of materials and x1, x2 and x3 be their respective thickness. Assume hot face and cold face temperatures as T1 and T2 respectively with neat sketch. 						          	          (7)

(OR)

17.	a.	A furnace is constructed with 225mm thick of fire brick, 120mm of insulating brick and 	225 mm of the building brick. The inside temperature is 1200 K and the outside 	temperature is 	330K. Find the heat loss per unit area and the temperature of the junction of 	the fire brick and insulation brick .

Data:  	Thermal conductivity of fire brick= 1.4 (W/m K), Thermal conductivity of insulating brick= 0.2 (W/m K) and Thermal conductivity of building brick= 0.7 (W/m K).	          (8)

b.	Derive an express on hallow  sphere with neat sketch.				          (7)

18.	Calculate the heat transfer coefficient for water at 60 ºC , flowing through a tube of i.d 0.625 cm with a velocity is 0.9 m/s.Tube wall temperature is 40 ºC .At the mean  temperature ( ie 50 ºC ),

data given: Viscosity of water is2.177 kg/ m.hr,

K= 0.54 kcal/ m.hr. ºC and Cp =1 kcal/kg.K,

density = 1000 kg/ m3

(OR)

19.	Derive the empirical equation for calculation of heat transfer coefficient in

a.	Laminar condition.									           (8)

b.	Turbulent condition in forced convection.					   	           (7)

20.	a.	Crude oil flows at the rate of 1000kg/hr through the inside pipe of a double pipe heat 	exchanger and is heated from 30 ºC to 90 ºC. The heat is supplied by kerosene initially 	at 	200 ºC 	flow through the annular space. If the temperature 	of approach (minimum 	temperature 	difference) is 10 ºC, Determine the heat transfer area for co current flow 	and kerosene flow rate.

Data: Specific heat of crude oil=0.5kcal/ kg ºC, Cp of kerosene is 0.6 kcal/ kg  ºC and Uo= 400 kcal/ hr m2 ºC.									           (7)

b.	Discuss about design of 1-1 shell and tube heat exchanger with neat sketch. 	           (8)

(OR)

21.	a.	Derive an express for single effect evaporator with neat Sketch.			           (5)

b.	A single effect evaporator is fed with 10000 kg/ hr of solution containing 10% solution by weight. Feed temperature is 303K and is to be concentrated to a solution of 20% solute by weight. The evaporator is at atmospheric pressure (101.325 k pa) and area of evaporatoris138m2.Saturated steam is supplied at 143.3 k pa as a heating medium. Calculate steam economy and overall heat transfer coefficient.

Data:

Enthalpy of feed at 303K = 125.79 kJ/ kg,

Enthalpy of vapour at 101.325 kpa= 2676.1 kJ/ kg,

Enthalpy of saturated steam at 143.325 kpa= 2691.5 kJ/kg, Enthalpy of product= 419.1 kJ/kg,

Enthalpy of saturated water at383K= 461.30  kJ/kg, Saturation temperature of steam= 383K

Boiling point of solution = boiling point water= 373K                                		         (10)

22.	a.	What is a vortex? How will you prevent the vortex? 					          (7)

b.	Derive the power calculation in mixing.						          (8)

(OR)

23.	Give an account of the different types of impellers and their characteristics with neat sketch.

24.	Describe the top basket centrifugal filtration with neat sketch? With neat sketch and give its merit and demerits.

(OR)

25.	A plate and frame press filtering slurry, gave a total of 25m3 of filtrate in 30 minutes and 35 m3 in 60 minutes when filtration was stopped. Estimate the washing time in minutes if 10 m3 of wash water used. The resistance of the cloth can be neglected and a constant pressure is used throughout.