UNIVERSIT       UNIVERSITY

Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec - 2016

| Code : 09BT222 / 12BT220 / BT210 |  | Duration : | 3 hrs |
| --- | --- | --- | --- |
| Sub. Name : DOWNSTREAM PROCESSING | MassTransfes | Max.Marks | 100 |

(Extra ordinary graph allowed)

| Q. No. | Answer All Questions |  | Marks |
| --- | --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |  |
| 1. | List out the destabilizing factors for a bio product? |  | 1 |
| 2. | The cell disruption are done by the physical methods. |  | 1 |
| 3. | What are filter aid and give its uses? |  | 1 |
| 4. | What is the use of centrifugation? |  | 1 |
| 5. | What is the main difference between adsorption and absorption? |  | 1 |
| 6. | Define permeation. |  | 1 |
| 7. | What is retention volume VR? |  | 1 |
| 8. | What is the basis behind the use of gel filtration? |  | 1 |
| 9. | The reason for polishing a biopharmaceutical is to achieve the________. |  | 1 |
| 10. | What is freeze drying? |  | 1 |
| PART B(5 X 3= 15 MARKS) |  |  |  |
| 11. | With a suitable example define inter cellular and intra cellular products. |  | 3 |
| 12. | How are centrifuges classified? |  | 3 |
| 13. | Draw the line diagram for an aqueous two phase extraction process. |  | 3 |
| 14. | List out the three chromatographic techniques. |  | 3 |
| 15. | Define Wet bulb temperature and dry bulb temperature? |  | 3 |
| PART C(5 X 15= 75 MARKS) |  |  |  |
| 16. | a. | Discuss about the physical and rheological characteristics of the bioproducts. | 7 |
|  | b. | Write down the downstream steps involved in citric acid production. | 8 |
| (OR) |  |  |  |
| 17. |  | Describe the various methods of cell disruption in detail. | 15 |
| 18. | a. | Give an account on the working of a continuous rotary drum filter with neat sketch and give its merit and demerits. | 8 |
|  | b. | The specific resistance of the cake of biomass was found to vary with pressure drop as follows. Find the compressibility of the cake. Pressure drop(KN/m2); 330; 134.3; 46.1; 21.1. Cake resistance (m/kg); 3.56 X 1011; 2.16 X 1011; 1.45 X 1011; 1.07 X 1011. | 7 |
| (OR) |  |  |  |
| 19. | a. | Explain the operation of a disc bowl centrifuge with neat sketch. | 7 |
|  | b. | i) Calculate the maximum velocity at which spherical particles of silica of 0.05 mm in diameter will fall through still water of temperature 75ºC. Data,Spgravity silica = 2.70,Sp .gravity of water =1.0 and viscosity of water= 0.30 Cp ii) Calculate the terminal velocity of a steel ball, 2 mm diameter ( density =7870 kg/m3) in oil. Density = 900 kg/m3), Viscosity 50 mNS/M2.. | 4+4 |
| 20. | a. | Explain the basic principles of super critical fluid extraction with a neat sketch | 7 |
|  | b. | Enumerate the various advantages of the super critical fluid extraction. | 8 |
| (OR) |  |  |  |
| 21. | a. | The partition coefficient of an organic acid in an organic solvent and water system is 2.7. i.Calculate the volume of organic solvent required to extract 99% of the acid from the 50 ml of aqueous solution. ii.How many extractions with 50 ml of organic solvent would be required to extract 99 % of the acid? | 7 |
|  | b. | Classify the various membrane separation processes and explain any one in detail. | 8 |
| 22. |  | What are the basic principles of size exclusion chromatography? Describe in detail. | 15 |
| (OR) |  |  |  |
| 23. | a. | Two proteins of molecular weights 2.5 x 105 and 1 x 104 were eluted out of a gel filtration column at 220 ml and 300 ml respectively. Determine the molecular weight of the protein which elutes at 270 ml from the column under the same conditions. | 6 |
|  | b. | Write a detailed note on hydrophobic interaction chromatography. | 9 |
| 24. |  | Explain the rate of drying as a function of free moisture contents for a hygroscopic material. | 15 |
| (OR) |  |  |  |
| 25. |  | Write short notes on any three: a. Crystallization. b. Equipments of crystallization. c. Types of various driers and their function. d. Batch freeze drier. | 3x5 |

ALL THE BEST