UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 09CE212/12CE211/CE247 | Duration : | 3 hrs |
| Sub. Name : | GEOTECHNICAL ENGINEERING/ SOIL MECHANICS/ MECHANICS OF SOILS | Max. marks : | 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |
| 1. | Define Void ratio. | 1 |
| 2. | What is the degree of saturation for a fully saturated soil? | 1 |
| 3. | Define coefficient of permeability. | 1 |
| 4. | State Darcy’s law. | 1 |
| 5. | What is meant by consolidation? | 1 |
| 6. | What is a flow net? | 1 |
| 7. | What is optimum moisture content? | 1 |
| 8. | Write down the Mohr’s-Coulomb failure envelope equation. | 1 |
| 9. | List the lab tests for determining Shear strength parameters. | 1 |
| 10. | Define friction circle. | 1 |

| PART B(5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11 | Classify soil according to its size. | 3 |
| 12 | What are the factors affecting permeability? | 3 |
| 13 | What do you understand by piping? | 3 |
| 14 | Differentiate between compaction and consolidation. | 3 |
| 15 | What are the causes of settlements? | 3 |

| PART C(5 X 15= 75 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 16. |  | A soil sample in its undisturbed state was found to have a volume of 105cm3 and mass of 201g. After oven drying the mass reduced to 168g. Compute water content void ratio, porosity, degree of saturation and air content. Take G=2.7. | 15 |
| (OR) |  |  |  |
| 17. | a. | Derive relationship between void ratio, water content, specific gravity and degree of Saturation. | 8 |
|  | b. | Define liquid limit, plastic limit, shrinkage limit. What are its uses? | 7 |
| 18. |  | List and explain the methods to determine the co-efficient of permeability in lab with a neat sketch. | 15 |
| (OR) |  |  |  |
| 19. | a. | The following data were recorded in a constant head permeability test. Internal diameter of permeameter = 7.5 cm Head loss over a sample length of 18 cm= 24.7 cm Quantity of water collected in 60s = 626 ml Porosity of the soil sample was 44% Calculate the coefficient of permeability of the soil. | 10 |
|  | b. | Define particle size distribution curve and list its uses in soil engineering. | 5 |
| 20. |  | A soil deposit of 12 m thickness overlies an impermeable stratum. The void ratio of the soil is 0.78 and G is 2.65. The water table is at a depth of 3.5 m below the ground surface. The zone of capillary saturation is 1.5 m above the water table and the degree of saturation of the soil above the capillary zone is55%. Draw the diagrams showing the variation of total stress, neutral stress and effective stress. | 15 |
| (OR) |  |  |  |
| 21. |  | State the assumptions made in Terzaghi’s one dimensional consolidation theory. Explain spring analogy method in detail. | 15 |
| 22. | a. | Explain different laboratory compaction methods. | 7 |
|  | b. | A concentrated load of 100 kN acts on the surface of homogenous soil mass of large extent. Find vertical pressure at depth of 5 m depth (i) directly under load (ii) at a horizontal distance of 3 m. | 8 |
| (OR) |  |  |  |
| 23. | a. | A concentrated load of 1000 kN is applied at the ground surface. Determine the vertical stress at a point P which is 6 m directly below the load. Also calculate the vertical stress at a point R which is at a depth of 6 m but at a horizontal distance of 5 m from the axis of the load. | 10 |
|  | b. | What are the factors that affect compaction? | 5 |
| 24. | a. | Briefly explain the measurement of shear strength of soil using triaxial test. | 5 |
|  | b. | Explain how shear strength parameters can be obtained from direct shear test. What are the limitations of this test? | 10 |
| (OR) |  |  |  |
| 25. |  | Explain friction circle method in detail. | 15 |

ALL THE BEST