Reg.No. _____________

UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 09CE221/12CE219/CE248 | Duration : | 3 hrs |
| Sub. Name : | FOUNDATION ENGINEERING | Max. marks : | 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A (10X1=10 MARKS) |  |  |
| 1. | What is SPT number? | (1) |
| 2. | What is meant by ‘ultimate bearing capacity’ of soil? | (1) |
| 3. | How would you decide the depth of exploration? | (1) |
| 4. | Define: allowable bearing pressure. | (1) |
| 5. | State True/False: Bearing capacity of clay is dependent on size of the footing. | (1) |
| 6. | What is meant by ‘negative skin friction’? | (1) |
| 7. | What do you mean by ‘passive earth pressure’? | (1) |
| 8. | Define end bearing pile. | (1) |
| 9. | State the function of a well curb. | (1) |
| 10. | State the situations under which mat foundation is adopted. | (1) |

| PART-B (5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11 | Explain the different types of foundation settlements? | (3) |
| 12 | What are the assumptions made in Terzaghi’s bearing capacity theory? | (3) |
| 13 | What are the different classifications of pile foundations? | (3) |
| 14 | How do you check the stability of retaining wall? | (3) |
| 15 | Explain the factors to be considered while designing the thickness of well steining? | (3) |

| PART-C (5 X 15= 75 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 16. |  | A 2m x 2m footing is located at a depth of 1.5m from ground surface in dense sand. The relative shear parameters, cohesion c = 0, angle of internal friction  = 36. Determine the ultimate bearing capacity of the soil if, a. water table is well below the foundation level. b. water table is at the surface. c. water table is at the base of footing. Take the unit weight of soil as 18 kN/m3 and saturated unit weight of soil as 20kN/m3. | (5) (5) (5) |
| (OR) |  |  |  |
| 17. | a. | Define soil exploration. What are the objectives of soil exploration? | (7) |
|  | b. | Briefly explain with neat sketches Standard Penetration Test and the corrections to be applied to find the N value. | (8) |
| 18. | a. | What are the factors influencing bearing capacity of a soil? Explain. | (5) |
|  | b. | A plate load test was conducted on a uniform deposit of sand and the following results were obtained. The size of the plate is 750mm x 750mm and the test is conducted at 1.5m below the ground. Pressure(KPa); 0; 50; 100; 200; 300; 400; 500; 600. Settlement(mm); 0; 1; 2; 4; 7.5; 12.5; 20; 40. A square footing 2m x 2m is to be founded at the same depth where the plate load test is conducted. Determine the allowable load on the footing so that the factor of safety against shear failure is not less than 3. | (10) |
| (OR) |  |  |  |
| 19. |  | Calculate the safe bearing capacity of a rectangular footing 2m x 4m in plan founded at a depth of 1.5m below the ground surface. The load on the footing acts at angle of 15 degrees to the vertical and is eccentric in the direction of width by 15cm. The saturated unit weight of the soil is 18kN/ m3. The rate of loading is slow and hence effective stress parameters can be used in the analysis. C’=15KN/m2 and φ’=25 degrees. Natural water depth is 2m below the ground level. Use IS: 6403- 1981 Recommendations. | (15) |
| 20. | a. | Define the term coffer dam. What are the different types? | (9) |
|  | b. | What is the safe bearing capacity of a single pile if the total penetration for the last 5 blows was observed to be 2.5 cm while driving a pile by drop hammer weighing 5kN with a height of drop equal to 80cm? | (6) |
| (OR) |  |  |  |
| 21. |  | In a 25 pile group, the pile diameter is 40 cm and centre to centre spacing of the square group is 1.25 m. If the cohesion between the pile and the soil along the surface is 45kN/m2, determine whether the failure would occur with the pile action individually or as a group? Neglect bearing at the tip of the pile. All the piles are 12 m long. The shear mobilization factor around each pile is 0.7. | (15) |
| 22. | a. | How piles are classified based on material, mode of transfer of load, construction and uses? Explain. | (8) |
|  | b. | Explain ‘Negative skin friction’ in detail. | (7) |
| (OR) |  |  |  |
| 23. | a. | Explain the different types of piers and caissons. | (15) |
| 24. | a. | How are shallow foundations classified? | (8) |
|  | b. | Explain core cutter method in detail. | (7) |
| (OR) |  |  |  |
| 25. |  | With the aid of neat diagram explain the principal components of a well foundation? What are the forces acting on it? | (15) |

ALL THE BEST