Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	09EC224						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give an example for multichannel signal.

2.	List two applications of FFT algorithms.

3.	What is Gibb’s phenomenon?

4.	What are the advantages of FIR filter?

5.	What is aliasing?

6.	Write any one property of Chebyshev type –1 filters.

7.	What are the different formats of fixed point representation?

8.	Define Noise Transfer Function (NTF).

9.	List any two digital signal processing application with TMS 320 family.

10.	What are the disadvantages of Harvard architecture?

PART – B  (5 x 3 = 15 MARKS)

11.	Compare overlap add and overlap save method of sectioned convolutions.

12.	What are the advantages and disadvantages of Digital filters?

13.	Compare the Butterworth and Chebyshev Type-1 filters.

14.	What is meant by product quantization error?

15.	What are four phases available in pipeline technique?

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Determine the 4-point DFT of the given data sequence xn=13,13,13.             		    (5)

b. 	Perform circular convolution of the two sequences x 1(n) and x2(n), where,

x1(n)= {0.2,0.4,0.6,0.8,1.0,1.2,1.4,1.6} and x2(n)= {0.1,0.3,0.5,0.7,0.9, 1.1,1.3, 1.9}     (10)

(OR)

17.	Compute eight point DFT of a sequence x (n) = {1, 2, 3, 4, 4, 3, 2, 1} using DIT algorithm.

18.	a.	What is meant by Optimum equiripple design criterion? Why it is followed?       		    (4)

b.	Write the procedure for FIR filter design by frequency sampling method             		    (4)

c.	Obtain the cascade realization of system function H z=1+2z−1−z−21+z−1−z−2    (7)

(OR)

19.	Design a linear phase FIR high pass filter  using hamming window, with a cut off frequency, ωc= 0.8Π rad/sample and N=7.

20.	Design a digital Butterworth Low pass filter with the following specification

0.6≤Hejω≤1, 0≤ω≤0.35πHejω ≤0.1, 0.7π≤ω≤π

with T=0.1 sec. Apply bilinear transformation.

[P.T.O]

(OR)

21. 	a.	Explain the design procedure involved in designing a low pass digital Chebyshev filter. (5)

b.	For the analog transfer function,  Has=2s+1s+2, determine H (z) if (i) T=1 sec and

(ii) T=0.1 sec  using bilinear transformation                                                                     (10)

22.	In the IIR system given below the products are rounded to 4-bits (including sign bit)

HZ=11−0.35z−11−0.62z−1. Find the output roundoff noise power in direct form realization.

(OR)

23.	a. 	Explain finite word length effects in realization of digital filters.                                     (10)

b. 	Write short notes:   (i) Truncation (ii) Rounding                                                                 (5)

24.	Discuss about adaptive noise cancellation using LMS algorithm in detail.

(OR)

25.	Draw and describe the features of Van Neumann and Harvard architecture.