Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

Subject Title :	NETWORKS & PROTOCOLS FOR INSTRUMENTATION & CONTROL													      	     				     Time : 3 hours

Subject Code:	09EI226								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A __________ System is designed for sending data in both directions.

2.	The number of bits transmitted per second is defined as __________.

3.	OSI reference model has __________ layers.

4.	RS232 is a point to point serial interface standard. True/False

5.	__________ Serial interface standard operates in tri-state.

6.	What is simplex transmission?

7.	STX in BSC protocol means __________.

8.	A router is a __________.

9.	An __________ address is provided by the network layer.

10.	Data can be exchanged in two modes in Modbus. They are __________ and __________.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the basic function of Physical layer?

12.	How is ARQ flow control algorithm different from simple stop and wait approach?.

13.	Compare RS232 and RS485.

14.	Draw the response frame of MODBUS protocol for reading a register of address 000B  with content 11AB .

15.	What are the layers in TCP/IP architecture?

PART – C  (5 x 15 = 75 MARKS)

16.	Describe in detail with necessary diagrams, how UART functions as a transmitter and receiver?

(OR)

17.	a.	Discuss in detail about any two data coding methods. 			               			    (5)

b.	Explain with examples simplex and duplex communication.          					    (5)

c.	What is a protocol? How is it different from standards?							    (5)

18.	Explain in detail about the most commonly used parallel interface standard IEEE 488.

(OR)

19.	Explain the architecture of USB with the reference to OSI layers.

20.	Describe the bit oriented protocol in Data-link layer that uses stop and wait AQQ approach for flow control.

(OR)

21.	Discuss the functions of individual layers of OSI reference model for communication between open systems	.

22.	a.	Draw the flow chart for software logic of Transmitter in Allan Bradley Data highway plus 	protocol.															       			    (5)

b.	Discuss the architecture of field bus communication protocol.                          		  (10)

(OR)

23.	Elaborate on the industrial protocol that superimposes digital information over the existing 4mA to 20mA current loop

24.	Discuss in detail about the IEEE802.3 LAN standard.

(OR)

25.	a.	Explain the different controlled media access control mechanisms.	           			  (10)

b.	Differentiate circuit switching ad packet switching    			                   			    (5)