Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec - 2016

Subject Title :	SIGNALS AND SYSTEMS								      Time : 3 hours

Subject Code:	09IT207 / 12IT201 / IT240				 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Discrete Time Signal with an example.

2.	Is y(t) = x2(t) system static or dynamic?

3.	Mathematically represent convolution sum.

4.	What is meant by natural response of the system?

5.	How the non-ideal filters are represented?

6.	Fourier transform of (t) is _______.

7.	State sampling theorem.

8.	Laplace transform of unit step signal is _______.

9.	Define system function.

10.	ROC of Z[(n)] is _______.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate energy signal and power signal. Give mathematical representations.

12.	Given x[n] = {2, -1, 2, 3, 4}. Represent x[n] as a linear combination of weighted shifted impulse functions.

13.	How the magnitude and phase are represented for the frequency response of LTI system?

14.	A signal having spectrum ranging from 10Hz to 20KHz is to be sampled and converted into discrete form. What is its Nyquist rate and Nyquist interval?

15.	Find Z-Transform and its ROC of x(n)={1,-2,6,4,7,-8}

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Given, xn= 1,2,3,4−1↑,0,2,1,−2   plot the following signals.

i.	x(3n)

ii.	x(n+2)

iii.	x(2n)

iv.	2x(n1)																  (12)

b.	Find the odd and even components of the signal x(t)=sin(t)+2sin(t)+2sin2(t)cos(t).																					    (3)

(OR)

17.	a.	Verify whether the system y(t) = t3 x(t2) is linear, time invariant, causal and static system.

(7)

b.	List the classification of signals based on their characteristics and explain each one with an 	example.																	    (8)

18.	Find the output of the system with impulse response h(t)=e−5tu(t)and the input signal

x(t)=[u(t3) u(t5)] .

(OR)

[P.T.O]

19.	A LTI system is characterized by the differential equation d2y(t)dt2+4dy(t)dt+4y(t)=dx(t)dt+x(t). Solve this equation if the initial conditions are dy(0+)dt=5;y(0+)=94and if the input is e−3tu(t).

20.	Using mathematical representations, explain first order and second order discrete time systems.

(OR)

21.	a.	List the classification of discrete time filters based on their frequency domain 					characteristics. 	Brief the ideal magnitude characteristics of each filter.			    (9)

b.	State and prove any three properties of continuous time Fourier transform.			    (6)

22.	Explain impulse train sampling and reconstruction of signal from its samples with neat diagrams.

(OR)

23.	Realize the system described by the differential equation using Direct form I and Direct form II.

d3y(t)dt3+4d2y(t)dt2+7dy(t)dt+8y(t)=5d2x(t)dt2+4dx(t)dt+7x(t)

24.	Using long division method, find the inverse Z-Transform of X(Z)=Z+1(2−3Z+Z2).when x(n) is causal and anticausal.

(OR)

25.	a.	Determine the impulse response and frequency response of the given linear constant 	coefficient difference equation y(n)−16y(n−1)−16y(n−2)=x(n).				  (10)

b.	Prove Convolution property using Z-Transform.									    (5)