Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec - 2016

Subject Title :	ENGINEERING MECHANICS						      	     Time : 3 hours

Subject Code:	09ME201/12ME230				 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define Engineering Mechanics.

2. 	What is meant by coplanar forces?

3. 	_______ is only applied for three coplanar concurrent forces, which are in equilibrium.

4. 	State varignon’s theorm.

5. 	Write the centroid formula for circle	.

6. 	Define Centre of gravity.

7. 	The motion of a particle along a straight line is known as rectilinear motion. (True/False)

8. 	Momentum =

9. 	The ratio of limiting friction to the normal reaction is known as _________.

10. 	Describe the dynamic friction.

PART – B  (5 x 3 = 15 MARKS)

11. Two concurrent forces of 12N and 18N acting at an angle of 60˚ .Find the resultant force.

12. 	A simply supported overhanging beam 20m long carries a system of loads and a couple as shown  in fig. Determine the reactions at support A and B

13. 	Write the expression to find a centroid of a composite plane figure.

14. 	A stone is dropped from the top of a tower, reaches the ground in 8 seconds .find

(a) the height of the tower (b) velocity of the particle, when it reaches the ground.

15.	A wheel of weight 1000N and diameter 600mm is required to move on a horizontal surface. If the co-efficient of rolling resistance is 15mm, calculate the force required to roll the wheel without slipping.

PART – C  (5 x 15 = 75 MARKS)

16.	A force vector of magnitude 100N ,is represented by a line AB of co-ordinates A(1,2,3) and B(5,8,12) Determine (a) the components of force along x,y and z axis (b) angles with x,y and z axis (c) specify the force vector.

(OR)

17.	The resultant of the force system shown infig .is 520N along the negative direction of the Y-axis. Determine the P and Ɵ.

[P.T.O.]

18.	A system of parallel forces are acting on rigid bar as shown in fig. Reduce the system to

(a) single force    (b) a single force and a couple at A       (c) a single force and couple at B

(OR)

19.	Find the reactions at the supports A and B, of the beam as shown in the fig.

20.	Find the moment of inertia of the section shown in the figure

(OR)

21.	Find the moment of inertia of the section shown in the figure, about its horizontal centroidal axis.

22.   Two blocks of weight 150 N and 50 N are connected by a string and passing over a frictionless 	pulley, as shown in fig. Determine the velocity of 150N  block after 4 seconds .Also calculate 	the  tension in the string.

(OR)

23.	Two blocks of weight 150 N and 50 N are connected by a string and passing through over a frictionless pulley, as shown in fig. Determine the acceleration of blocks A and B and the tension in the string.

24. 	A 7m long ladder rests against a vertical wall, with which it makes an angle of 45˚ and on a floor .If a man whose weight is one half that of the ladder climbs it, at what distance along the ladder will he be, when the ladder is about to slip? Take co-efficient of friction between the ladder and the wall is 1/3 and that between the ladder and the floor is 1/2.

(OR)

25.	In a screw jack, the pitch of the square threaded screw is 5.5 mm and the mean diameter is

70 mm. The force exerted in turning the screw is applied at the end of a lever 210 mm long measured from the axis of the screw. If the co-efficient of friction of the screw jack is 0.07, calculate the force required at the end of the lever to (a) raise a weight of 30 kN (b)	lower the same weight.