UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 09ME202/ME228 | Duration : | 3 hrs |
| Sub. Name : | ENGINEERING THERMODYNAMICS | Max. marks : | 100 |

| Q. No. | Questions |  | Marks |
| --- | --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |  |
| 1. | Distinguish between open and closed system. |  | (1) |
| 2. | Define Zeroth law of thermodynamics. |  | (1) |
| 3. | What is perpetual motion machine of the second kind? |  | (1) |
| 4. | State Carnot theorem? |  | (1) |
| 5. | What is meant by quality of steam? |  | (1) |
| 6. | Define triple point for a pure substance. |  | (1) |
| 7. | State Avogadros law. |  | (1) |
| 8. | What is compressibility factor? |  | (1) |
| 9. | State the assumptions made in air standard cycles. |  | (1) |
| 10. | Define mean effective pressure. |  | (1) |
| PART B(5 X 3= 15 MARKS) |  |  |  |
| 11. | Prove that energy of an isolated system is always constant. |  | (3) |
| 12. | State the Kevin-Planck and Clausius statement of the second law of thermodynamics. |  | (3) |
| 13. | One kg of steam at 10 bar has an enthalpy of 2500kJ/kg. Find its quality. |  | (3) |
| 14. | What are assumption made for the ideal gas equation? |  | (3) |
| 15. | Sketch the P-V and T-S diagram for a Brayton Cycle? |  | (3) |
| PART C(5 X 15= 75 MARKS) |  |  |  |
| 16. |  | A piston and cylinder machine contains a fluid system which passes through a complete cycle of four processes. During a cycle, the sum of all heat transfers is -170kJ. The system completes 100 cycles per minute. Complete the following table showing the method for each item, and compute the net rate of work output in kW. Process; Q(kJ/min); W(kJ/min); DE(kJ/min). a-b; 0; 2,170; -------. b-c; 21,000; 0; -------. c-d; -2,100; -------; -36,600. d-a; ----------; -------; -------. | 15 |
| (OR) |  |  |  |
| 17. |  | Derive the steady flow energy equation and reduced it to i) Nozzle ii) Air Compressor iii) Condenser and list down the set of assumptions for each case. | 15 |
| 18. |  | A Carnot engines receives 90kJ from a reservoir at 627°C. It rejects heat to the environment at 27°C. One –fifth of its work output is used to drive a Carnot refrigerator. The refrigerator rejects 60kJ to the environment at 27°C. Find i)the work output of the engine ii) the efficiency of the heat engine iii) temperature of the low temperature reservoir for the refrigerator in degree Celsius iv) the COP of the refrigerator. | 15 |
| (OR) |  |  |  |
| 19. | a. | Establish the inequality of Clausius. | 8 |
|  | b. | One kg of ice at -5°C is exposed to the atmosphere which is at 20°C. The ice melts and comes into thermal equilibrium with the atmosphere. Determine the entropy increase of the universe. | 7 |
| 20. |  | Draw and explain the phase equilibrium diagram for a pure substance on T-S coordinates with relevant constant property lines. | 15 |
| (OR) |  |  |  |
| 21. |  | Steam at 0.8 MPa , 250°C and flowing at the rate of 1kg/s passes into a pipe carrying wet steam at 0.8MPa, 0.95 dry. After adiabatic mixing the flow rate is 2.3kg/s. Determine the condition of steam after mixing. The mixture is now expanded in a frictionless nozzle isentropically to a pressure of 0.4MPa. Determine the velocity of the steam leaving the nozzle. Neglect the velocity of steam in the pipe line. | 15 |
| 22. |  | Explain with a neat sketch the working of a Bomb calorimeter. | 15 |
| (OR) |  |  |  |
| 23. |  | A vessel of volume 0.3m3 contains 15kg of air at 303K. Determine the pressure exerted by the air using a) Perfect gas equation b)Vander Waals equation c) Generalized compressibility chart. The critical temperature of air is 132.8 K and critical pressure of air is 37.7 bar. | 15 |
| 24. |  | An engine working on the Otto cycle is supplied with air at 0.1 MPa , 35°C. The compression ratio is 8. Heat supplied is 2100kJ/kg. Calculate the maximum pressure and temperature of the cycle, the cycle efficiency and the mean effective pressure (For air Cp=1.005, Cv=0.718 and R=0.287kJ/kgK). | 15 |
| (OR) |  |  |  |
| 25. |  | Dry saturated steam is supplied to a steam turbine at 12 bar and after the expansion its condenser pressure is 1 bar. Find the Rankine cycle efficiency, specific steam consumption, Neglect the feed pump work. | 15 |

ALL THE BEST