UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  |  |  | Semester : | 2016-17 ODD |  |
| --- | --- | --- | --- | --- | --- | --- |
| Code : |  |  | 09ME203/ME229 | Duration : | 3 hrs |  |
| Sub. Name : |  |  | Thermal Engineering | Max. marks : | 100 |  |
| Q. No. | Questions |  |  |  |  | Marks |
| PART-A(10X1=10 MARKS) |  |  |  |  |  |  |
| 1. | In a two stroke cycle engine, the four operations namely suction, compression, expansion and exhaust are completed in _____________ number of crankshaft revolution(s). |  |  |  |  | (1) |
| 2. | Increasing the compression ratio in S.I engines increases the tendency for knocking. True or False. |  |  |  |  | (1) |
| 3. | The ratio of exit to inlet pressure for maximum flow rate per unit area of steam through a nozzle when steam is initially dry and saturated is ___________________________ . |  |  |  |  | (1) |
| 4. | In steam turbines, the reheat factor ________________ with increase in number of stages. |  |  |  |  | (1) |
| 5. | The clearance volume in reciprocating air compressors is provided to __________________. |  |  |  |  | (1) |
| 6. | The efficiency of a vane type air compressor as compared to roots air compressor for the same pressure ratio is ___________________ . |  |  |  |  | (1) |
| 7. | The COP of a Carnot Refrigerator is ____________________ than that of a Carnot heat pump. |  |  |  |  | (1) |
| 8. | ________________________ efficiency is defined as the ratio of actual volume of gas drawn into the compressor to the piston displacement. |  |  |  |  | (1) |
| 9. | In unsaturated air, the water vapour exists in the _________________ state. |  |  |  |  | (1) |
| 10. | The relative humidity of air during cooling and humidification increases. True or False. |  |  |  |  | (1) |
| PART B(5 X 3= 15 MARKS) |  |  |  |  |  |  |
| 11. | Differentiate flash point and fire point. |  |  |  |  | (3) |
| 12. | State the reasons for the reduction in final velocity of steam flowing through a nozzle. |  |  |  |  | (3) |
| 13. | State 3 advantages of multi-stage compression over single stage compression. |  |  |  |  | (3) |
| 14. | What is the effect of superheating on the COP of a vapour compression system ? Justify. |  |  |  |  | (3) |
| 15. | What is sensible heating ? Represent the process on a Psychrometric chart. |  |  |  |  | (3) |
| PART C(5 X 15= 75 MARKS) |  |  |  |  |  |  |
| 16. | a. | Explain the ideal and actual valve timing diagrams of a four-stroke petrol engine with neat sketch. |  |  |  | (10) |
|  | b. | Write short note on the ignition delay period in C.I engine combustion. |  |  |  | (5) |
| (OR) |  |  |  |  |  |  |
| 17. | a. | Explain the principle and working of a Magneto ignition system with a sketch. |  |  |  | (10) |
|  | b. | What are the effects of detonation? How is detonation controlled? |  |  |  | (5) |
| 18. |  | Steam initially dry and saturated is expanded in a nozzle from 15 bar at 300C to 1.0 bar. If the frictional loss in the nozzle is 12% of the total heat drop, calculate the mass of steam discharged when the exit diameter of the nozzle is 15 mm. |  |  |  | (15) |
| (OR) |  |  |  |  |  |  |
| 19. |  | Explain the construction and working of a pressure compounding of steam turbine with the help of a neat sketch. Also indicate the change in velocity and pressure as steam passes through the compounding stages. |  |  |  | (15) |
| 20. | a. | Derive the condition for minimum work done in a multi-stage compression. |  |  |  | (10) |
|  | b. | Explain the methods used to achieve isothermal compression in high speed compressors. |  |  |  | (5) |
| (OR) |  |  |  |  |  |  |
| 21. |  | A two-cylinder single acting air compressor is to deliver 16 kg of air per minute at 7 bar from suction conditions 1 bar and 15C. Clearance may be taken as 4% of stroke volume and the index for both compression and re-expansion as 1.3. Compressor is directly coupled to a four-cylinder four stroke petrol engine which runs at 2000 rpm with a brake mean effective pressure of 5.5 bar. Assuming a stroke : bore ratio of 1.2 for both engine and compressor and a mechanical efficiency of 82% for compressor, calculate the required cylinder dimensions. |  |  |  | (15) |
| 22. |  | A simple vapour compression plant produces 5 tonnes of refrigeration. The enthalpy values at the inlet to compressor, exit from compressor and at the exit from condenser are 183.19, 209.41 and 74.59 respectively. Estimate (a) refrigerant flow rate (b) COP (c) power required to drive the compressor (d) rate of heat rejection to the condenser. |  |  |  | (15) |
| (OR) |  |  |  |  |  |  |
| 23. |  | Explain the construction and working of a vapour absorption refrigeration system with a sketch. |  |  |  | (15) |
| 24. | a. | A sling psychrometer in a laboratory test recorded the following readings. Dry Bulb Temperature (DBT) = 35C Wet Bulb Temperature (WBT) = 25C Calculate (i) Specific Humidity (ii) Relative Humidity (iii) Vapour density in air (iv) Dew point temperature (v) Enthalpy of mixture per kg of dry air. Take atmospheric pressure as 1.0132 bar. |  |  |  | (10) |
|  | b. | Explain the process of heating and dehumidification with a line diagram and represent the process on a psychrometric chart. |  |  |  | (5) |
| (OR) |  |  |  |  |  |  |
| 25. |  | It is required to design an air conditioning plant for a small office room for the following winter condition : Outdoor conditions = 14C DBT and 10C WBT Required conditions = 20C DBT and 60% RH Amount of air circulation = 0.3 m3/min/person Seating capacity of the office = 60 The required condition is first achieved by heating and then by adiabatic humidifying. Determine the following (i) heating capacity of the coil in kW and the surface temperature required if the by pass factor of coil is 0.4. (ii) Capacity of humidifier. |  |  |  | (15) |

ALL THE BEST