Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December - 2016

Subject Title: HEAT AND MASS TRANSFER      						      Time: 3 hours

Subject Code: 09ME204	/12ME217/ME230					       	       Maximum Marks: 100

#### Answer ALL questions

| Q. No | Question PART – A (10 x 1 = 10 MARKS) |  |  | Mark |
| --- | --- | --- | --- | --- |
| 1. | The thermal conductivity is expressed as ______________. |  |  | 1 |
| 2. | Heat conduction in gases is due to elastic impact of _________________. |  |  | 1 |
| 3. | In graphical method ___________________ is defined as the ratio of number of flow lanes / tubes divided by the number of temperature increments. |  |  | 1 |
| 4. | Define Biot number. |  |  | 1 |
| 5. | Write the dimensionless parameters used in forced convection |  |  | 1 |
| 6. | Define Fins. |  |  | 1 |
| 7. | Define molar concentration. |  |  | 1 |
| 8. | Compared to parallel flow heat exchanger LMTD in case of counter flow heat exchanger will be______________. |  |  | 1 |
| 9. | Define emissivity? |  |  | 1 |
| 10. | Define Radiation? |  |  | 1 |
|  | PART – B (5 x 3 = 15 MARKS) |  |  |  |
| 11. | Write down the assumptions made in the calculations of heat flow in a composite (multilayer) wall. |  |  | 3 |
| 12. | What are the assumptions for lumped heat analysis? |  |  | 3 |
| 13. | Define Fin effectiveness. |  |  | 3 |
| 14. | In a double pipe heat exchanger, hot fluid with a specific heat of 2300 J/kg K enters at 380˚C and leaves at 300˚C.Cold fluid enters at 25˚C and leaves at 210˚C.Calculate the heat exchanger area required for counter flow. Take U=750 W/m2K and mh =1 kg/s |  |  | 3 |
| 15. | Assuming the sun to be a black body emitting radiation maximum intensity at λ = 0.49μm,calculate the following : (i) The surface temperature of the sun (ii) the heat flux at surface of the sun |  |  | 3 |
|  | PART – C (5 x 15 = 75 MARKS) |  |  |  |
| 16. | Derive the general heat conduction equation in cylindrical coordinates. |  |  | 15 |
|  |  |  | (OR) |  |
| 17. | Air at 90˚C flows in a copper tube of 5 cm inner diameter with thermal conductivity 380 W/mK and with 0.7 cm thick wall which is heated from the outside by water at 120˚C.A scale of 0.4 cm thick is deposited on the outer surface of the tube whose thermal conductivity is 1.82W/mK.The air and water side unit surface conductances are 220 W/m2K and 3650 W/m2K respectively. Calculate (i) Overall water to air transmittance. (ii) Water to air heat exchange. (iii) Temperature drop across the scale deposit. |  |  | 15 |
| 18. | An egg with mean diameter of 40 mm and initially at 20˚C is placed in a boiling water pan for 4 minutes and found to be boiled to the consumer’s taste. For how long should a similar egg for same consumer be boiled when taken from a refrigerator at 5˚C.Take the following properties of egg (k = 10 W/m˚C, ρ = 1200 kg/m3 , c = 2 kJ/kg˚C and h = 100 W/m2˚C).Use lump theory |  |  | 15 |
|  |  |  | (OR) |  |
| 19. | A pipe 500 mm in diameter carrying steam at a temperature of 160 ο C ( for heating the houses in winter) and another pipe 150 mm diameter carrying water at 11 ο C (for supplying drinking water) are buried under the ground with centre to centre distance 1.5 m. The length of each pipe is 60 m and thermal conductivity of soil is 0.3 W/m ο C. Assuming that the pipes run parallel to each other, the earth is infinite and resistance of the pipe materials is negligible, determine (i) the net heat transfer rate between the two pipes (ii) the rise in water temperature due to heat transfer if the velocity of the water in the pipe is 2m/min. |  |  | 15 |
| 20. | Air at 200C and 1atm pressure is flowing over a flat plate at a velocity of 3m/s.If plate is 280 mm wide and at 560C, Calculate the following quantities at x =280 mm: (i) Boundary layer thickness (ii) Local friction coefficient (iii) average friction coefficient (iv)Shearing stress due to friction (v)Thermal boundary layer thickness (vi) Local convective heat transfer coefficient (vii) Average convective heat transfer coefficient (viii) Rate of heat transfer by convection (ix) Total mass flow rate through the boundary (x)Total drag force on the plate. |  |  | 15 |
|  | (OR) |  |  |  |
| 21. | Ten thin brass fins (k = 100 W/mK),0.75 mm thick are placed axially on a 1m long and 60 mm diameter engine cylinder which is surrounded by 270C.The fins are extended 1.5 cm from the cylinder surface and the heat transfer co-efficient between cylinder and atmospheric air is 15 W/m2K.Calculate the rate of heat transfer and the temperature at the end of fins when the cylinder surface is at 1600C.(Assume fin end as insulated) |  |  | 15 |
| 22. | a. | Derive an expression for LMTD of a parallel-flow heat exchanger. |  | 8 |
|  | b. | A counter flow heat exchanger is to heat air entering at 4000C with a flow rate of 6 kg/s by the exhaust gas entering at 8000C with a flow rate of 4 kg/s.The overall heat transfer coefficient is 100 W/m2K and the outlet temperature of the air is 551.50C.Specific heat at constant pressure for both air and exhaust gas can be taken as 1100 J/kgK. Calculate (i) The heat transfer area needed. (ii) The number of transfer units. |  | 7 |
|  | (OR) |  |  |  |
| 23. | a. | Two large tanks, maintained at the same temperature and pressure are connected by a circular 0.15 m diameter direct which is 3 m in length. One tank contains a uniform mixture of 60 mole % ammonia and 40 mole % air and the other tank contains a uniform mixture of 20 mole % ammonia and 80 mole % air. The system is at 273 K and 1.013 × 105 pa. Determine the rate of ammonia transfer between two tanks. Assuming a steady state mass transfer |  | 8 |
|  | b. | Air at 200C and atmospheric pressure, containing small quantities of iodine flows with a velocity of 4 m/s inside a 4 cm diameter tube. Determine the mass transfer coefficient. Assume Dab = 0.75 × 10-5 m2/s. |  | 7 |
| 24. | a. | Water is to be boiled at atmospheric pressure in a polished copper pan by means of an electric heater. The diameter of the pan is 0.38 m and is kept at 1150C.Calculate the following (i) power required to boil the water (ii) rate of evaporation (iii) critical heat flux. |  | 7 |
|  | b. | A condenser is to be designed to condense 600 kg/h of dry saturated steam at a pressure of 0.12 bar. A square array of 400 tubes, each of 8 mm diameters is to be used. The tube surface is maintained at 300C .Calculate the heat transfer co-efficient and the length of each tube. |  | 8 |
|  | (OR) |  |  |  |
| 25. | a. | Calculate the following for an industrial furnace in the form of a black body and emitting radiation at 25000C: 1. Monochromatic emissive power at 1.2μm length. 2. Wavelength at which the emission is maximum. 3. Total emissive power , and 4. Total emissive power with Ɛ = 0.9 |  | 8 |
|  | b. | Two very large parallel plates with emissivities 0.5 exchange heat. Determine the percentage reduction in the heat transfer rate if a polished aluminium radiation shield of Ɛ = 0.04 is placed in between the plates. |  | 7 |