UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 09ME207/ ME217 | Duration : | 3 hrs |
| Sub. Name : | Gas Dynamics and Jet Propulsion | Max. marks : | 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |
| 1. | Define Mach number. | (1) |
| 2. | Define compressible flow. | (1) |
| 3. | State the function of nozzle. | (1) |
| 4. | In the divergent part of a nozzle, the Mach number is ______than unity. | (1) |
| 5. | Define Rayleigh flow. | (1) |
| 6. | Give two practical examples for Fanno flow. | (1) |
| 7. | Define oblique shock wave. | (1) |
| 8. | Shock waves develop in supersonic flow [True/False] | (1) |
| 9. | Classify Jet propulsion. | (1) |
| 10. | Specific impulse is the ratio between ______and _______. | (1) |

| PART B(5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11 | An air stream at p=1 bar, T=400 K and c=400 m/s is brought to rest adiabatically. Determine the stagnation temperature. | (3) |
| 12 | Draw the Temperature – entropy diagram for compression process. | (3) |
| 13 | Define Fanno flow. State the assumptions used in the analysis of Fanno flow. | (3) |
| 14 | Name the governing equations for the Normal shock process. | (3) |
| 15 | List the advantages of Ram Jet Engine. | (3) |

| PART C(5 X 15= 75 MARKS) |  |  |
| --- | --- | --- |
| 16. | Argon is stored in a reservoir at 300 K; determine stagnation enthalpy and velocity of sound in it. γ=1.658, and the molecular weight of argon is 39.94. | (15) |
| (OR) |  |  |
| 17. | An air jet (γ=1.4, R=287J/kgK) at 400 K has sonic velocity. Determine i) Velocity of sound at 400 K, ii) Velocity of sound at stagnation condition, iii) Maximum velocity of jet, iv) Stagnation enthalpy and v) Crocco number. | (15) |
| 18. | A conical diffuser has entry and exit diameters of 15 cm and 30 cm respectively. The pressure, temperature and velocity of air at entry are 0.69 bar, 340 K and 180 m/s respectively. Determine a) The exit pressure b) The exit velocity and c) The force exerted on the diffuser walls. Assume isentropic flow , γ =1.4, Cp = 1.00 KJ/kg K. | (15) |
| (OR) |  |  |
| 19. | A nozzle in a wind tunnel gives a test section Mach number of 2. Air enters the nozzle from a large reservoir at 0.69 bar and 310K. The cross sectional area of the throat is 1000 cm2. Determine the following quantities for the tunnel for one dimensional isentropic flow: i) Pressure, temperature and velocity at the throat and test sections, ii) Area of cross-section of the test section, iii) Mass flow rate, iv) Power required to drive the compressor. | (15) |
| 20. | Air at P0=10 bar, T0=400 K is supplied to a 50 mm diameter pipe. The friction factor for the pipe surface is 0.002. If the Mach number changes from 3 at the entry to 1 at the exit determine i) The length of the pipe and ii) The mass flow rate. | (15) |
| (OR) |  |  |
| 21. | The conditions of a gas in a combustor at entry are: P1=0.343 bar,T1 =310 K C1=60 m/s. Determine the Mach number, pressure, temperature and velocity at the exit if the increase in stagnation enthalpy of the gas between entry and exit is 1172.5 kJ/kg. Take Cp=1.005kJ/kg K, γ = 1.4. | (15) |
| 22. | Derive the Prandtl- Meyer relation, starting from the energy equation for flow through a normal shock. | (15) |
| (OR) |  |  |
| 23. | The state of a gas (γ=1.3, R=469 J/kg K) upstream of a normal shock wave is given by the following data: Mx=2.5, Px=2 bar, Tx=275 K Calculate the Mach number, pressure, temperature and velocity of the gas downstream of the shock. | (15) |
| 24. | Explain the working principle of Turbojet engine with a neat sketch. | (15) |
| (OR) |  |  |
| 25. | Explain the working principle of solid propellant rockets with a neat sketch. | (15) |

ALL THE BEST