UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 09ME228/ ME223 | Duration : | 3 hrs |
| Sub. Name : | Heat Engines and Fluid Machinery | Max. marks : | 100 |

| Q. No. | Questions | Marks |
| --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |
| 1. | Define fluid density. | (1) |
| 2. | Define viscosity. | (1) |
| 3. | Classify hydraulic turbines according to the type of energy at inlet. | (1) |
| 4. | Name any two types of draft tubes. | (1) |
| 5. | Define closed system. | (1) |
| 6. | Entropy change depends on _______. | (1) |
| 7. | The difference between indicated power and brake power is ______. | (1) |
| 8. | The constant volume cycle is also called as ______. | (1) |
| 9. | State the Fourier law of heat conduction. | (1) |
| 10. | Define thermal conductivity. | (1) |

| PART B(5 X 3= 15 MARKS) |  |  |
| --- | --- | --- |
| 11 | Distinguish between a single acting and double acting reciprocating pump. | (3) |
| 12 | Write short notes on governing of turbines. | (3) |
| 13 | State the Kelvin Planck’s statement of second law of thermodynamics. | (3) |
| 14 | Draw the P-V and T-s diagram of Brayton cycle. | (3) |
| 15 | The surface temperature of a central heating radiator is 333 K. What is the radiation heat transfer between radiator and surroundings at 293 K? | (3) |

| PART C(5 X 15= 75 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 16. | Explain the working principle of a centrifugal pump with a neat sketch. |  | (15) |
| (OR) |  |  |  |
| 17. | Calculate the density, specific weight and weight of 1 litre of petrol of specific gravity=0.7. |  | (15) |
| 18. | A Pelton wheel is having a mean bucket diameter of 1m and is running at 1000 rpm. The net head on the pelton wheel is 700 m. If the side clearance angle is 150 and discharge through nozzle is 0.1 m3/s, find : - Power available at the nozzle, and - Hydraulic efficiency of the turbine. |  | (15) |
| (OR) |  |  |  |
| 19. | Explain the working of an axial flow reaction turbine (Kaplan turbine) with a neat sketch. |  | (15) |
| 20. | A quantity of gas occupying 0.14 m3 at a pressure of 1400 kPa and 3000C is expanded isentropically to 280 kPa. Calculate (i) mass of gas, (ii) final temperature and (iii) Work transfer. Take Cp=1.04 kJ/kg K, Cv=0.74 kJ/kg K, R=0.3 kJ/kg K. |  | (15) |
| (OR) |  |  |  |
| 21. | The cyclic heat engine operates between a source temperature of 800˚C and sink temperature of 30˚C. What is the least rate of heat rejection per kW net output of the engine? |  | (15) |
| 22. | Derive an expression for thermal efficiency of Diesel cycle. |  | (15) |
| (OR) |  |  |  |
| 23. | The pressure and temperature of air at the beginning of compression in an Otto cycle is 103 kPa and 270C, respectively. The heat added per kg of air is 1850 kJ. The compression ratio is 8. Determine maximum temperature, maximum pressure and thermal efficiency. |  | (15) |
| 24. | a. | The wall of a furnace is constructed from 15 cm thick fire brick having constant thermal conductivity of 1.7 W/mK. The two sides of the walls are maintained at 1400 K and 1150 K, respectively. What is the rate of heat loss through the wall which is 0.5 m X 3 m on a side? | (8) |
|  | b. | Hot air at 1500C flows over a flat plate maintained at 500C. The forced convection heat transfer coefficient is 75 W/m2K .Calculate the heat gain rate by the plate through and area of 2 m2. | (7) |
| (OR) |  |  |  |
| 25. | Write short notes on the following modes of heat transfer. (i) Conduction, (ii) Convection, (iii) Radiation |  | (15) |

ALL THE BEST