UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec – 2016

|  |  | Semester: | 2016-17 ODD |
| --- | --- | --- | --- |
| Code: | 09ME237/12ME216 | Duration: | 3 hrs |
| Sub. Name: | POWER PLANT ENGINEERING | Max. Marks: | 100 |

| PART-A(10X1=10 MARKS) |  |  |  |
| --- | --- | --- | --- |
| 1. | Regenerative cycle in steam power plant is used to _________ thermal efficiency. |  | (1) |
| 2. | Name the two fluids used in binary vapour cycle. |  | (1) |
| 3. | Economizer is used to heat ________. |  | (1) |
| 4. | The draught which a chimney produces is called _________ draught. |  | (1) |
| 5. | State the function of a moderator in a nuclear reactor. |  | (1) |
| 6. | A closed cycle gas turbine works on _______ cycle. |  | (1) |
| 7. | The ratio of brake power to indicated power of an I.C. engine is called _______. |  | (1) |
| 8. | State the function of a surge tank. |  | (1) |
| 9. | Define load factor of a power station. |  | (1) |
| 10. | State the function of a solar collector. |  | (1) |
| PART B(5 X 3= 15 MARKS) |  |  |  |
| 11. | Draw the T-s diagram of a reheat cycle. |  | (3) |
| 12. | Classify surface condensers. |  | (3) |
| 13. | Write the disadvantages of gas turbine over I.C. Engine. |  | (3) |
| 14. | Name the main components of a diesel engine power plant. |  | (3) |
| 15. | Write short notes on MHD system. |  | (3) |
| PART C(5 X 15= 75 MARKS) |  |  |  |
| 16. | In a steam turbine steam at 20 bar, 3600C is expanded to 0.08 bar. It then enters a condenser, where it is condensed to saturated liquid water. The pump feeds back the water into the boiler. Assume ideal processes; find per kg of steam the net work and the cycle efficiency. |  | (15) |
| (OR) |  |  |  |
| 17. |  | Explain with a neat diagram the working of a Binary vapour cycle. | (15) |
| 18. |  | Explain pneumatic ash handling system with a neat sketch. | (15) |
| (OR) |  |  |  |
| 19. |  | Discuss with a neat layout, the working of a modern steam power plant. | (15) |
| 20. |  | With a neat sketch, explain the working of pressurized heavy water reactor. Write its advantages and disadvantages. | (15) |
| (OR) |  |  |  |
| 21. |  | How reheating can improve the thermal efficiency of an open cycle gas turbine plant. | (15) |
| 22. |  | Explain with the help of a neat sketch the working of a thermostatically controlled cooling system. | (15) |
| (OR) |  |  |  |
| 23. |  | Explain hydro electric power plant with a neat sketch. | (15) |
| 24. |  | A power station has to supply load on daily basis as follows: Time hours; 0 – 6; 6 - 12; 12-14; 14-18; 18-22; 22- 24. Load (MW); 30; 100; 60; 80; 100; 60. - Draw the load curve. - Draw the load duration curve. - Select suitable generation units to supply the loads. - Calculate the load factor. - Calculate the capacity of the plant and the plant capacity factor. | (15) |
| (OR) |  |  |  |
| 25. |  | With a neat sketch, explain the working of a wind power plant. Mention its advantages and disadvantages. | (15) |