UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 10AE202 | Duration : | 3 hrs |
| Sub. Name : | Basics of Aerospace Engineering | Max. marks : | 100 |

| Q. No. | Questions |  | Marks |
| --- | --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |  |
| 1. | Give an example for monoplane aircraft. |  | (1) |
| 2. | Mention the relation between angle of attack and lift. |  | (1) |
| 3. | What is meant by fly-by-wire system? |  | (1) |
| 4. | The outermost layer of the earth’s atmosphere__________. |  | (1) |
| 5. | What is the function of bulkhead? |  | (1) |
| 6. | Name any TWO structural components of an aircraft wing. |  | (1) |
| 7. | What is meant by stealth aircraft? |  | (1) |
| 8. | What is the significance of non-air breathing engine? |  | (1) |
| 9. | Calculate wing are for the rectangular wing with Aspect ratio=3 and wing span b=5m. |  | (1) |
| 10. | Expand HAL and VSSC. |  | (1) |
| PART B(5 X 3= 15 MARKS) |  |  |  |
| 11 | What are the features of ornithopter? |  | (3) |
| 12 | An airplane is flying at an altitude where the air temperature is 37˚C with a Mach number of 1.2. Determine the velocity of the airplane in km/h. |  | (3) |
| 13 | Define geometric altitude. |  | (3) |
| 14 | Mention any TWO applications of aluminium alloy in the aircraft. |  | (3) |
| 15 | What are the types of gas turbine engine? |  | (3) |
| PART C(5 X 15= 75 MARKS) |  |  |  |
| 16. | a. | Merits and demerits of biplane. | (8) |
|  | b. | Discuss the sources of four forces of flight. | (7) |
| (OR) |  |  |  |
| 17. |  | Discuss the developments in aerodynamics with suitable sketches. | (15) |
| 18. |  | The air temperature and pressure in the reservoir of the subsonic wind tunnel are To=1000K and Po=10atm, respectively.The static temperatures at the throat and exit are T*=833K and Te=300K, respectively. The mass flow through the nozzle is 0.5 kg/s. For air, specific heat at constant pressure Cp=1008J/kgK. Calculate - Velocity at the throat V* - Velocity at the exit Ve (iii ) Area at the throat A* | (5) (5) (5) |
| (OR) |  |  |  |
| 19. |  | Compare the performance of different types of airfoil with neat sketches. | (15) |
| 20. |  | Explain in detail about the classification of aircraft with neat sketches. | (15) |
| (OR) |  |  |  |
| 21. | a. | Write short notes on Aerodynamic centre | (3) |
|  | b. | Center of pressure | (3) |
|  | c. | Drag divergence mach number | (4) |
|  | d. | Pitching, yawing and rolling | (5) |
| 22. |  | Discuss the various structures of fuselage with their functions. | (15) |
| (OR) |  |  |  |
| 23. |  | Suggest the suitable materials for fuselage, wing, empennage and landing gear and also justify it. | (15) |
| 24. |  | Explain the working principle of turbofan engine with a neat sketch. | (15) |
| (OR) |  |  |  |
| 25. |  | Derive thrust equation for a rocket engine. | (15) |

ALL THE BEST