Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec - 2016

Subject Name:	ELECTRON DEVICES								      	     Time : 3 hours

Subject Code:	10EC201						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

- The mass of an electron is _____________.

- What is Magnetic deflection sensitivity of a CRT?

- Avalanche breakdown of PN junction diode occurs_______.

(a) in forward bias 		(b) in reverse bias 	(c) due to manufacturing defect

(d) due to decrease in temperature

- The mobility of an intrinsic semiconductor ________ with increase in temperature.

- Write the Diode equation.

- In which region the transistor will be operated as a switch?

- Draw the symbol of the following devices.

  - SCR  				b. DIAC

- What is the other name for Varactor diode?

- Photo diode is used in ___________ communication.

10.  List the advantages of Nano Crystal Displays.

PART – B (5 x 3 = 15 MARKS)

- Two plane parallel plates A and B are placed apart and potential of B is made 200 V positive with respect to plate A. An electron starts from rest from plate A. Calculate the velocity of the electron on reaching plate B.

- Differentiate between drift and diffusion currents.

- Define Miller’s theorem.

- Draw the equivalent circuit of tunnel diode and list applications.

- Write short notes on Optocoupler?

PART – C  (5 x 15 = 75 MARKS)

- Briefly discuss the following

a) Force in a Magnetic field.  	b) Motion in a Magnetic field.					   (7+8) (OR)

- Explain the working principle of a CRT with a neat diagram. List out the various applications of the CRO.

- Derive the continuity equation in semiconductor.

(OR)

- Write technical notes on:

a).	Transition Capacitance 												        (7)

b).	Diffusion Capacitance 												        (8)

- Explain the working principle of PN diode under forward and reverse biased conditions with its volt-ampere characteristics.

(OR)

- Derive the Eber-Moll’s Model Equation.

- a). With a neat diagram explain the working principle of SCR.   					       (10)

b). Draw and explain the two transistor model of an SCR.						         (5)

(OR)

- Explain in detail the construction, equivalent circuit, working and static characteristics of TRIAC. Compare DIAC and TRIAC.

- With neat diagrams, explain the construction and operation of photodiode and phototransistor.

(OR)

- Write short notes on the following:

a) Light Emitting Diode 		b) Liquid Crystal Display						     (8+7)