Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec - 2016

Subject Title :	ELECTRIC CIRCUITS AND NETWORKS			      	     Time : 3 hours

Subject Code:	10EE201/12EE201								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Kirchhoff’s current law.

2.	If a 200 Ω resistor is connected across the terminals of a 5 V battery what is the power dissipated in the resistor?

3.	When the superposition theorem is applied to any circuit, the dependent voltage source is always ___________.

4.	___________ Theorem permits to transfer source from one position to another.

5.	Define transient time.

6.	The maximum value of coefficient of coupling is ___________.

7.	A network function is given by Z(s)=s+4s. Find the pole-zero plot.

8.	In a balanced star connected system, line voltages are ___________ ahead of their respective phase voltages.

9.	Which parameters are widely used in transmission line theory?

10.	One decibel is equal to ________N.

PART – B  (5 x 3 = 15 MARKS)

11.	Convert the given star into an equivalent delta connection.

12.	State Maximum power transfer theorem.

13.	Define Low pass and High pass filter.

14.	Define dot convention.

15.	Two inductively coupled coils have self-inductances L1 = 50 mH and L2 = 200 mH. If the coefficient of coupling is 0.5, find the value of mutual inductance between the coils.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Determine the current delivered by the source in the circuits shown in the figure	.	  (10)

b.	List out the types of circuit elements.											    (5)

[P.T.O]

(OR)

17.	a.	Derive an expression of star to delta transformation in detail.						  (10)

b.	Sketch the dependent and independent sources.									    (5)

18.	a.	Find the current in the 6 Ω resistors using the principle of superposition for the circuit of 	figure.                            														  (11)

b.	State Superposition Theorem.													    (4)

(OR)

19.	With the help of suitable circuit, prove the Norton’s Theorem.

20.	Obtain the transient response of a RL series circuit with DC input and explain the significance of time constant.

(OR)

21.	In the figure, the switch K is closed at position 1 at t = 0. Assuming no initial current through inductor, Find instant of time at which voltage across R equals voltage across L.

22.	Derive an expression for output voltage, output current and amplification of single tuned circuit.

(OR)

23.	A three-phase delta connected RYB system with an effective voltage of 400 V, has a balanced load with impedances 3+j4Ω. Calculate the:

a.	phase currents  	b.	line currents

c.	power in each phase.

24.	Design the m-derived T and π section high pass filter with a cut off frequency of 10 KHz with design impedance of  500 ohms and m = 0.4.

(OR)

25.	a.	Define the term Neper and Decibel.												    (5)

b.	The impedance parameters of a two port network are

Z11 = 6 Ω, Z22 = 4 Ω, Z12 = Z21 = 3 Ω.

Compute the Y parameters.													  (10)