UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | 10EI214/12EI215/EI248 | Duration : | 3 hrs |
| Sub. Name : | Communication Engineering | Max. marks : | 100 |

| Q. No. | Questions |  | Marks |
| --- | --- | --- | --- |
| PART-A(10X1=10 MARKS) |  |  |  |
| 1. | Audio signals are preferably …………….. modulated |  | (1) |
| 2. | Define the term “bandwidth”. |  | (1) |
| 3. | Define modulation index of AM. |  | (1) |
| 4. | What is SNR if the signal of 15mW is corrupted by a noise of 0.5mW? |  | (1) |
| 5. | What is the advantage of balanced modulator over nonlinear modulation? |  | (1) |
| 6. | …………….. can be used as a source for Fiber optic communication. |  | (1) |
| 7. | Noise is measured in-- -- -- -- -- -- -- -- - |  | (1) |
| 8. | What is the advantage of Successive approximation method of ADC? |  | (1) |
| 9. | Hamming code is a method of error correction. State True/False. |  | (1) |
| 10. | What is the front porch of video information? |  | (1) |
| PART B(5 X 3= 15 MARKS) |  |  |  |
| 11. | What is the need for modulation? |  | (3) |
| 12. | An antenna transmits an AM signal hosing a total power content of 15KW. Determine the power being transmitted at the carrier frequency and at each of sidebands when the percentage of modulation is 85%. |  | (3) |
| 13. | What is DPCM? |  | (3) |
| 14. | List the advantages and disadvantages of TDM and FDM |  | (3) |
| 15. | Why is the shape of the cell hexagon in cellular communication? |  | (3) |
| PART C(5 X 15= 75 MARKS) |  |  |  |
| 16. | a. | Derive the mathematical analysis for AM signal and obtain the spectrum of AM. | (10) |
|  | b. | What is the depth of modulation of a amplitude modulated signal whose total transmitting power is 1000W and carrier power is 350W | (5) |
| (OR) |  |  |  |
| 17. | a. | Discuss the various types of modulation systems. | (5) |
|  | b. | Derive the expression for modulation index represented in terms of maximum and minimum voltage. | (10) |
| 18. | Explain the Balance modulator method of generating suppressed carrier AM signal. |  | (15) |
| (OR) |  |  |  |
| 19. | Discuss the FM detection using PLL. |  | (15) |
| 20. | Discuss in detail the digital modulation techniques. |  | (15) |
| (OR) |  |  |  |
| 21. | How is delta modulation superior to other digital modulation techniques explain in detail? |  | (15) |
| 22. | Write short notes an: a. Co-axial Cable b. Any two methods of ADC converters |  | (15) |
| (OR) |  |  |  |
| 23. | Explain any two fiber optic detectors in detail |  | (15) |
| 24. | How is any target detected using RADAR explain with necessary diagrams. |  | (15) |
| (OR) |  |  |  |
| 25. | Draw the block diagram of color TV receiver and explain in detail. |  | (15) |

ALL THE BEST