Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec 2016

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	10MT203						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Draw the Discrete-time unit impulse signal.

2.	What is a causal system?

3.	What is the need for FFT algorithm?

4.	Distinguish between DTFT and DFT

5.	Give the equation for the order N of a Butterworth filter.

6.	What are the demerits of Impulse Invariant technique?

7.	Mention any one characteristics of FIR filter.

8.	What is the condition to obtain a linear phase response?

9.	What are the two types of number representation?

10.	What are the two types of quantization employed in digital system?

PART – B  (5 x 3 = 15 MARKS)

11.	A discrete – time signal x(n) is defined as  x(n) = {1,2,-3,4,0,5}. Sketch x(n-2).

12.	Draw the DIT-FFT Butterfly diagram.

13.	What is frequency warping?

14.	What is Gibbs phenomenon?

15.	How can overflow limit cycles be eliminated?

PART – C  (5 x 15 = 75 MARKS)

16.	State sampling theorem. Explain the sampling process with a neat diagram.

(OR)

17.	a.	Find the response  of the DTLTI system whose input is

x(n) = 1  ; n = -2,0,1

= 2  ; n = -1

= 0 ; elsewhere

And impulse response is h(n) = { 1, -1,1,-1}										    (8)

b.	Test if the DTLTI systems given below are linear,causal							    (7)

i.	y(n) =x(n2 )

ii.  	y(n) = x2 (n)

18.	Find the DFT of  x(n) = {0,1,2,3,4,5,6,7}  using DIT -  FFT algorrithm.

(OR)

19.	a.	Explain the procedure for computing the IDFT using FFT algorithm.    				    (5)

b.	Find the IDFT using direct computation Y(k) = {1,0,1,0}							  (10)

20.	Design a digital Butterworth filter using Bilinear Transformation method with Pass band and stop band cut-off frequencies 800 rad/sec and 1800 rad/sec.

The pass band attenuation is -3 dB and stop band attenuation is -10 dB.

(OR)

[P.T.O.]

21.	Realize the difference equation using Direct form I and II

Y(n) = - 0.1 y ( n – 1) + 0.72 y (n – 2) + 0.7 x(n) – 0.252 x(n – 2)

22.	The  desired frequency response of a filter is given by

HD ( ejω)      	=    e- j2   		; - П/4 ≤    ω    ≤ П/4

=   0			;    П/4 ≤ |ω| ≤ П

Obtain the coefficients of the FIR filter using a rectangular window

(OR)

23.	Describe the design of FIR filters using frequency sampling method.

24.	Discuss in detail about the representation of numbers in registers.

(OR)

25.	Explain quantisation error in detail.