UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.  No. ________

End Semester Examination – Nov / Dec – 2016

Subject Title :	AERODYNAMICS									      	     Time : 3 hours

Subject Code:	11AE205/12AE204   			 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is unit of kinetic energy?

a.	kg/m3        	b.	N            		c.	Nm            		d.	kg m/s

2.	What is unit of pressure?

a.	kg/m3        	b.	N/m2            		c.	Nm            		d.	kg m/s

3.	Lines of constant potential function and lines of constant stream function are

a.	parallel      	b.	orthogonal   		c.	at an acute angle  d.	at an obtuse angle

4.	Euler equation is manifestation of

a.	Newton’s I law of motion    		b.	Newton’s II law of motion

c.	Newton’s III law of motion  		d.	First law of thermodynamics

5.	How do you get Rankine Oval?

6.	What is sink flow?

7.	What is doublet flow?

8.	Biot-Savart law relates vortex strength and position vector to

a.	induced velocity    			b.	density  		c.	pressure  	d.	temperature

9.	Kutta condition specifies condition for ______ at the trailing edge of the airfoil

a.	velocity 						b.	density  		c.	pressure 		d.	velocity potential

10.	According to thin airfoil theory, the aerodynamic centre is at

a.	leading edge       		b.	c/4        		c.	c/2         		d.	3c/4

PART – B  (5 x 3 = 15 MARKS)

11.	Write continuity equation for incompressible 3D flow in cylindrical polar coordinate system. 	Define the variables.

12.	Define Path line.

13.	Define Stream function.

14.	State Kutta-Joukowskii theorem.

15.	State Kelvin’s Circulation theorem.

PART – C  (5 x 15 = 75 MARKS)

16.	The velocity potential for a flow condition is given by

φ=Ax3−3xy2

a.	Find the velocity components for this flow.										    (6)

b.	Find the corresponding stream function Ψ.										    (9)

(OR)

17.	The stream function Ψ for a flow condition is given by

ψ=A3x2y−y3

a.	Find the velocity components for this flow .										    (6)

b.	Find the corresponding potential function φ.										    (9)

[P.T.O]

18.	For a velocity field, in cylindrical coordinate system, given by

ur, θ=Ccosθ+Λ2π r

vr,θ=−Csinθ

Where C and A are arbitrary non-zero constants.

a.	Does this satisfy continuity equation for incompressible flow?  					    (9)

b.	If yes, find the pressure distribution for satisfying the equation of motion.			    (6)

(OR)

19.	For a velocity field given by

ux,y=x2−2xycosy2

vx,y=−2xy+siny2

a.	Find out whether continuity equation is satisfied for incompressible flow or not.  	    (9)

b.	If continuity equation is satisfied, find the pressure distribution so as to satisfy the momentum equation.															    (6)

20.	For a combination of source and sink of the same strength Q at (-c,0) and (c,0) respectively, derive the Potential function Φ.

(OR)

21.	For a source with strength Q at (0,0) in a uniform stream with velocity U along x axis, find the stagnation points. Assume that stagnation points will be on the x-axis.

22.	Using Biot-Savart law, derive the expression for induced velocity due to straight line segment of vortex with strength Γ at a point which is at a distance h from it and making angle θ1 and θ2 at two ends of the straight line segment.

(OR)

23.	Find the velocity induced due to vortex of strength Γ in the form of a square with sides 2l at its centre.

24.	For a thin aerofoil, the velocity distribution on the upper and lower surface is given by uu=Axc1−xc andul=Axc2 1−xc respectively where c is chord length of aerofoil and origin is at leading edge. For free stream velocity U and density ρ, find the lift of aerofoil using Kutta-Joukowskii theorem.

(OR)

25.	Show that for elliptic distribution of circulation, the induced downwash velocity due to trailing vortex is constant along the span of the wing.