UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

Subject Title :	ELECTROMAGNETIC FIELDS						      	     Time : 3 hours

Subject Code:	11EE206/12EE202/EE248		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Convert point P(1,3,5) in to cylindrical coordinates.

2.	Define gradient of a scalar field.

3.	Draw the electric field produced by a positive charge.

4.	Write Poisson’s equation.

5.	Draw the magnetic field due to a straight current carrying conductor.

6.	Draw a solenoid inductance. Mark the various parameters.

7.	State Faraday’s Law.

8.	Define time-harmonic field.

9.	EM wave can travel in a perfect dielectric medium with a speed of light. True/False

10.	Write the expression for Poynting Vector.

PART – B  (5 x 3 = 15 MARKS)

11.	State Stokes’s theorem.

12.	Mention practical applications of Coulomb’s law.

13.	State Biot-Savart’s Law.

14.	A parallel-plate capacitor with plate area of 5cm2 and plate separation of 3mm has a voltage 50sin103t volts applied to its pates. Calculate the displacement current assuming ε=2ε0.

15.	Write the expressions for Electric and Magnetic Field of an Electromagnetic Wave. Mention the significance of exponential terms in the expressions.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Write the expressions for differential length, differential surface and differential volume of 	a cylindrical coordinate system. 												    (5)

b.	Transform the following vector to cylindrical coordinates.   (10)

(OR)

17.	a.	Mention the properties of curl of a vector field. 									    (7)

b.	Show that the divergence of the curl of any vector field is zero.					    (8)

18.	Derive the expression for Electric field Intensity due to a charge which is in the form of a circular ring.

(OR)

19.	a.	Deduce an expression for the capacitance of a spherical capacitor. 					   (10)

b.	Given .  Find the electric field intensity and volume charge density at

P(-3,1,2). Take 													    (5)

20.	a.	Derive the expression for magnetic field intensity due to a circular current loop.            (10)

b.	A circular loop located on  carries a direct current of 10A along . Determine   at (0, 0, 4). 													    	    (5)

(OR)                                                                [P.T.O]

21.	A rectangular current carrying coil is placed in a magnetic field. Find the expression for torque produced.

22.	a.	Mention the disadvantage of eddy current in electrical machines. Can we effectively utilize 	the eddy current for practical applications? Justify. 								    (5)

b.	Derive the relationship between conduction current and displacement current densities.

(10)

(OR)

23.	Derive Maxwell’s equations. Write the Maxwell’s equations for static electric field.

24.	a.	Derive the expression for wave parameters of a perfect dielectric. 					   (10)

b.	An electric field in free space is given by  V/m. Find i) the direction of wave propagation   ii) β    iii) time taken by the wave travel a distance of λ/2.

(5)

(OR)

25.	Derive the expression for skin depth.  Find the skin depth in a copper for a frequency of 104 MHz. The conductivity of copper is 5.8×107 S/m.