UNIVERSITY

(Karunya Institute of Technology & Sciences)

(Declared as Deemed-to-be University under Sec.3 of the UGC Act, 1956)

Reg.No. _____________

End Semester Examination – Nov/Dec - 2016

|  |  | Semester : | 2016-17 ODD |
| --- | --- | --- | --- |
| Code : | EE251/11EE209/12EE209 | Duration : | 3 hrs |
| Sub. Name: | INDUCTION AND SYNCHRONOUS MACHINES | Max. Marks: | 100 |

#### Answer ALL questions

| Q. No. | Questions PART-A (10x1=10 MARKS) |  | Marks |
| --- | --- | --- | --- |
| 1 | Induction motor is always run ___________ than synchronous motor. |  | 1 |
| 2 | Mention the types of rotor available in three phase induction motor. |  | 1 |
| 3 | Name the type of motor used in Table fan. |  | 1 |
| 4 | How can we reverse the rotation of 1-phase, split-phase motor? |  | 1 |
| 5 | At what speed, a 6 pole alternator is to be driven to obtain the emf of frequency 50 Hz. |  | 1 |
| 6 | What is the main purpose of parallel operation? |  | 1 |
| 7 | Synchronous motors operate on any desired power factor. (True / False). |  | 1 |
| 8 | The V-curves of a synchronous motor show the relation between __________ and __________. |  | 1 |
| 9 | Which test is conducted to find the values of Xd and Xq of an alternator? |  | 1 |
| 10 | Name the person who developed the two reaction theory. |  | 1 |
| PART-B (5x3=15 MARKS) |  |  |  |
| 11 | Define slip of induction motor. When induction motor is at rest, what is the value of slip? |  | 3 |
| 12 | Draw the equivalent circuit of a single phase induction motor. |  | 3 |
| 13 | Illustrate voltage regulation of Alternator and also give possible reason for voltage dip. |  | 3 |
| 14 | Write the expression for power and power angle relation of synchronous machines. |  | 3 |
| 15 | What is meant by hunting in synchronous machines? How can it be suppressed? |  | 3 |
| PART-C (5x15=75 MARKS) |  |  |  |
| 16 | a. | Explain the different speed control methods of phase wound & cage induction motor. | 10 |
|  | b. | List various losses in three phase induction motor. | 5 |
| (OR) |  |  |  |
| 17 |  | Explain the procedure for the construction of circle diagram of a three-phase induction motor and explain how you will obtain the following from the circle diagram: - Maximum Torque - Starting torque - Maximum output | 15 |
| 18 |  | Explain using suitable diagrams: - Split-phase induction motor - Capacitor start induction motor | 15 |
| (OR) |  |  |  |
| 19 |  | Using double filed revolving theory; explain why a single Phase induction motor is not self starting. | 15 |
| 20 |  | Explain in detail, EMF &MMF method of predetermining the voltage regulation of Alternator. | 15 |
| (OR) |  |  |  |
| 21 |  | Explain the principle of operation of an alternator. Derive expressions of emf equation of three phase Alternator. | 15 |
| 22 |  | What do you mean by synchronizing of alternators? State the conditions and then explain any two method of synchronizing. | 15 |
| (OR) |  |  |  |
| 23 |  | Explain the phasor diagram of salient pole alternator and also determine Xd and Xq using slip test. | 15 |
| 24 |  | Explain the principle of operation and methods of starting of Synchronous motors. | 15 |
| (OR) |  |  |  |
| 25 |  | Explain how V and inverter V curves are drawn, using suitable experimental setup. | 15 |