Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec – 2016

Subject Title :	POWER SYSTEM ANALYSIS						      	     Time : 3 hours

Subject Code:	11EE216 / 12EE216 / EE255		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Given: Base voltage = 11,000V and Base power = 5 MVA. What is the value of base impedance?

2.	1+a+a2+a3= ________.

3.	Number of tree in a branch is ________ number of node in the network.

4.	In type 1 modification of Z-bus building algorithm, off diagonal elements are always _______.

5.	One of the ________ bus is considered as slack bus in the power flow analysis

6.	What is the need of load flow study?

7.	If the fuel cost of the power generating unit  F=0.06P2+40P+120, the incremental fuel cost at 40 MW is given by________.

8.	Mention anyone method used to do the unit commitment problem.

9.	Shunt faults are mainly due to ________.

10.	Transient Limit is always ________ than the steady state limit.

PART – B  (5 x 3 = 15 MARKS)

11.	Prove that neutral current can flow only if zero –sequence current is present.

12.	What are the different types of unsymmetrical fault, which occur in a power system?

13.	Differentiate Gauss Seidal and Newton Raphson method in the load flow study.

14.	Write the co-ordination equation including losses and explain the terms involved.

15.	Define Voltage sag and swell in power system.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the p.u. impedance diagram for the power system shown below.  Neglect resistance and use a base of 100MVA, 220kV in 50 line.  The ratings of the generator, motor and transformer are:

Generator	:   40MVA, 25kV, X = 20%

Motor		:  50MVA, 11kV, X = 30%

 -    Transformer:  40MVA, 33/220kV, X = 15%

 -    Transformer:  30MVA, 11 / 220kV, X = 15%

(OR)

17. 	a.	Write down the significance of the per unit representation.			        			    (4)

b.	Determine the positive sequence components of the given unbalanced three phase currents:

Ia=100 A   Ib= 120 A and Ic= 10130A. 					        				    (7)

c.	Brief about power system planning and operation	.			        				    (4)

[P.T.O]

18.	Derive the fault current expression for the following faults occurs in a power system.

a.	Single line to Ground Fault. 						       							    (7)

b.	Line to Line fault.							       								    (8)

(OR)

19.	Obtain bus impedance matrix using Z-Bus building algorithm for the following network.

20.	Explain the step by step computational procedure for the Newton Raphson  method  in load flow studies with appropriate expression.

(OR)

21.	a.	Derive the Load flow equations in polar form and Rectangle form.		        		    (8)

b.	Draw the flow chart of Gauss Seidal method used in load flow studies.		                   (7)

22.	a.	Derive the expression for loss formula co-efficient. 								    (8)                                               b.	Brief about unit commitment in power system			                                       	    (7)

(OR)

23.	a.	Write short notes on Hydro Thermal Scheduling				                           		    (8)

b.	A power plant has three units with the following fuel cost equations:

Maximum and minimum loading on each unit is 150 MW and 40 MW, the demand is 275MW.  Find the optimal scheduling.                                                                     	    (7)

24.	a.	Explain the Runge kutta method of analyzing multi machine power system for    stability 	with a neat flow chart.                      					      				  		  (10)

b.	Brief  about the reason for Voltage and frequency fluctuation. 	      	                    		   (5)

(OR)

25.	a.	Write Short Notes on Equal Area Criteria (EAC)  used in power system stability.		    (8)

b.	Derive the expression for swing equation in electrical machines. 	   	       		   	    (7)