Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – Nov/Dec 2016

Subject Title :	LINEAR INTEGRATED CIRCUITS					      	     Time : 3 hours

Subject Code:	11EE220/ 12EE221/ EE258						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the ideal characteristics of op-Amp?

2.	Define Slew rate.

3.	Which operational amplifier circuit is preferred for the gain below 100% ?

4.	What is the output voltage when a differentiator is fed with 10 V DC?

5.	Draw an inverting comparator Circuit.

6.	What is the other name of astable multivibrator?

7.	What is PLL? State any one of the application.

8.	What fixed voltage regulator IC is ideal for (-12V) regulation?

9.	Which ADC gives the instantaneous digital output?

10.	Draw any MOSFET switch configuration used in ADC.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the pin configurations of IC 741.

12.	State the difference between active filter and passive filters.

13.	Draw the circuit diagram of a fundamental log amplifier and write the output expression.

14.	State the merits of voltage regulator IC 723 over Fixed voltage regulators.

15.	What are the advantages of inverted R-2R ladder DAC over R-2R ladder type DAC?

PART – C  (5 x 15 = 75 MARKS)

16.	Write short notes on:

a.	Pole-zero compensation.														    (8)

b.	Dominant pole compensation.													    (7)

(OR)

17.	Discuss the following Dc characterizes of the Operational Amplifier

a.	Input bias current.															    (8)

b.	Input offset voltage.															    (7)

18.	What is the necessity of higher order filters? Derive the transfer function of the 2nd order low pass filter.

(OR)

19.	a. 	Write short notes on Integrators.												    (8)

b.	Design a 2 input Summing amplifier with gain of 2 for the first input and a gain of 3 for the second input.																    (7)

20.	With necessary diagrams and expressions, explain the exponential amplifier.

(OR)

21.	Explain the Full wave precision rectifiers with necessary diagrams and waveforms.

[P.T.O.]

22.	With neat circuit configurations, explain the astable multivibrator using IC 555.

(OR)

23.	Explain the functionality of a low voltage IC regulator -723 with short circuit protection.

24.	Explain the following with necessary diagrams.

a.	Flash type DAC.																    (7)

b.	Successive approximation type ADC.											    (8)

(OR)

25.	Explain the R-2R ladder type ADC and discuss its merits and demerits.