Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November 2016

Subject Title :	MICROPROCESSORS AND MICROCONTROLLERS   	     Time : 3 hours

Subject Code:	11EE221 / 12EE223 / EE263						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Which interrupt has the highest priority in 8085?

2.	List the flags of 8085 microprocessor.

3.	The 8051 has _________ bytes of on chip RAM.

4.	Write any four features of 8051 microcontroller.

5.	Identify the addressing mode used by each of the following instruction.

MOV A, #03H  and MOV A,@R0

6.	What is the value of register A after the execution of the following Instructions?

MOV A, #26H

SWAP A

7.	Quote the significance of C/T bit in TMOD register.

8.	What is the difference between timer and counter?

9.	Expand SOC and EOC.

10.	What is the purpose of 8255 PPI?

PART – B  (5 x 3 = 15 MARKS)

11.	What is program counter? How it is useful in program execution?

12.	Compare Microcontroller and Microprocessor.

13.	Explain Rotate Instructions.

14.	List the interrupts of Microcontroller according to its priority.

15.	What is the significance of EA pin?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	List different system buses of 8085 microprocessor and give function of each bus.	    (5)

b.	Draw the timing diagram for the instruction MVI B, Data.							  (10)

(OR)

17.	With a neat functional block diagram, explain the architecture of 8085 Microprocessor.

18.	Explain the following terms related with 8051:

a.	Program Status word.															    (5)

b.	Organization of internal RAM.													  (10)

(OR)

19.	Explain the architecture of 8051 Microcontroller with a neat functional block diagram,

20.	a.	Show the status of the CY, AC and P flag after the addition of 38H and 20H in the 	following instructions.

MOV A, #38H

ADD A, #20H																    (5)

b.	Explain different addressing modes of 8051 with suitable examples.				  (10)

(OR)

[P.T.O]

21.	What are the different groups of instructions supported by 8051? Explain them with suitable examples.

22.	a.	Discuss about various Bit handling instructions of 8051. 							    (5)

b.	Describe the serial communication feature of 8051.								  (10)

(OR)

23.	Explain four modes of operation of Timer of 8051 microcontroller.

24.	Write short notes on:

a.	Interfacing Digital to Analog Converter with 8051.								    (7)

b.	Interfacing External Memory with 8051.										    (8)

(OR)

25.	Draw the circuit diagram to interface stepper motor with 8051 microcontroller. Write a program to rotate the stepper motor in the clockwise direction.